SCHEMA DESIGN

CAUSAL MAPPING

Event 1: Link Flap

  - Physical & Environmental

    - Damaged Fiber Optic Cable

MECHANISM:

- High attenuation → Optical power below threshold (link remains UP)

SYSTEM STATE:

- Low optical signal with active interface

CAUSAL RELATIONSHIPS:

[Low Optical Power] → [CRC/FCS Errors]

[Low Optical Power] → [Packet Loss (0.1–5%)]

[Low Optical Power] → [TCP Retransmissions]

[Low Optical Power] → [Voice Jitter]

[Low Optical Power] → [Increased MTTR]

[Low Optical Power] → [Monitoring Blind Spots]

IMPACT GROUPING:

• Network Stability:

- CRC/FCS errors

- Occasional MAC table flush

• Service Disruption:

- Packet loss

- TCP retransmissions

- Voice degradation (jitter)

• Operational Impact:

- Increased MTTR

- OTDR testing required

- Vendor dependency

• Security Implications:

- NetFlow sampling gaps

    - Faulty SFP/Transceiver

MECHANISM:

- Voltage / temperature out of specification → Transceiver instability causing intermittent resets

SYSTEM STATE:

- Unstable optical transceiver with intermittent link resets and signal degradation

CAUSAL RELATIONSHIPS:

[Unstable Transceiver] → [Interface Reset / Flap]
[Unstable Transceiver] → [CRC/FCS Errors]
[Unstable Transceiver] → [LACP Renegotiation]
[Unstable Transceiver] → [Packet Loss (1–10%)]
[Unstable Transceiver] → [TCP Retransmissions]
[Unstable Transceiver] → [Increased MTTR]
[Unstable Transceiver] → [Monitoring Blind Spots]

IMPACT GROUPING:

• Network Stability:

- Interface resets / flapping

- CRC/FCS errors

- LACP state changes

• Service Disruption:

- Packet loss

- TCP retransmissions

- Intermittent connectivity

• Operational Impact:

- Increased MTTR

- SFP replacement / RMA required

- Vendor support engagement

• Security Implications:

- Monitoring gaps during link instability

- Reduced visibility in traffic analysis

    - Loose / Corroded Connections

MECHANISM:

- Poor physical contact / increased resistance → Intermittent signal disruption and micro-disconnections

SYSTEM STATE:

- Unstable electrical/optical continuity with transient link interruptions and degraded signal quality

CAUSAL RELATIONSHIPS:

[Intermittent Signal Loss] → [Interface Flapping]
[Intermittent Signal Loss] → [Carrier Transitions Spike]
[Intermittent Signal Loss] → [CRC/FCS Errors]
[Intermittent Signal Loss] → [Packet Loss (0.5–5%)]
[Intermittent Signal Loss] → [Voice Call Drops / Jitter]
[Intermittent Signal Loss] → [Increased MTTR]
[Intermittent Signal Loss] → [Monitoring Blind Spots]

IMPACT GROUPING:

• Network Stability:

- Interface up/down cycling

- Carrier transition spikes

- CRC/FCS errors

• Service Disruption:

- Packet loss

- Voice/video call drops

- Application timeouts

• Operational Impact:

- Increased MTTR

- Difficult fault isolation

- Repeated manual inspection of connectors

• Security Implications:

- Monitoring blind spots

- Potential packet inspection bypass

    - Power Supply Fluctuations

MECHANISM:

- Voltage instability (brownout / spike) → Device components reset or behave unpredictably causing interface flaps

SYSTEM STATE:

- Unstable power delivery leading to intermittent device or port resets

without complete shutdown

CAUSAL RELATIONSHIPS:

[Voltage Instability] → [Interface Flapping]
[Voltage Instability] → [Device / Module Reset]
[Voltage Instability] → [FHRP Failover]
[Voltage Instability] → [Packet Loss (10–100% during events)]
[Voltage Instability] → [Application Timeouts]
[Voltage Instability] → [Increased MTTR]
[Voltage Instability] → [Firewall State Inconsistency]

IMPACT GROUPING:

• Network Stability:

- Multiple interface flaps

- Device or module resets

- FHRP failover events

• Service Disruption:

- High packet loss during fluctuation

- Application timeouts

- Temporary connectivity loss

• Operational Impact:

- Increased MTTR

- Emergency troubleshooting effort

- Power infrastructure validation required

• Security Implications:

- Firewall/session state inconsistencies

- Temporary exposure during instability

    - Environmental (Temp / Humidity)

MECHANISM:

- Temperature / humidity out of operating range → Component instability and signal degradation causing intermittent link behavior

SYSTEM STATE:

- Environment-induced hardware stress leading to fluctuating interface performance and periodic link failures

CAUSAL RELATIONSHIPS:

[Environmental Stress] → [Interface Flapping]
[Environmental Stress] → [Transceiver / ASIC Errors]
[Environmental Stress] → [CRC/FCS Errors]
[Environmental Stress] → [Packet Loss (1–10%)]
[Environmental Stress] → [Latency / Jitter Increase]
[Environmental Stress] → [Increased MTTR]
[Environmental Stress] → [Monitoring Blind Spots]

IMPACT GROUPING:

• Network Stability:

- Interface instability / flapping

- Hardware-induced errors

- CRC/FCS increments

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Intermittent connectivity issues

• Operational Impact:

- Increased MTTR

- Need for environmental monitoring and control

- Repeated troubleshooting during peak conditions

• Security Implications:

- Monitoring gaps during failure periods

- Reduced visibility in network telemetry

    - Electromagnetic Interference

MECHANISM:

- External electromagnetic noise coupling into transmission medium → Signal corruption leading to increased bit error rate and intermittent link instability

SYSTEM STATE:

- Degraded signal integrity with elevated noise levels causing sporadic errors without complete link failure

CAUSAL RELATIONSHIPS:

[Signal Noise / EMI] → [CRC/FCS Errors]
[Signal Noise / EMI] → [Interface Flapping]
[Signal Noise / EMI] → [Packet Loss (0.5–5%)]
[Signal Noise / EMI] → [TCP Retransmissions]
[Signal Noise / EMI] → [Latency / Jitter Increase]
[Signal Noise / EMI] → [Increased MTTR]
[Signal Noise / EMI] → [Monitoring Blind Spots]

IMPACT GROUPING:

• Network Stability:

- CRC/FCS error increase

- Intermittent interface instability

• Service Disruption:

- Packet loss

- TCP retransmissions

- Increased latency / jitter

• Operational Impact:

- Increased MTTR

- Difficult root cause isolation (environmental source tracing)

- Need for shielding / cable rerouting

• Security Implications:

- Monitoring gaps due to corrupted or dropped packets

- Reduced effectiveness of traffic analysis systems

    - Patch Panel Degradation

MECHANISM:

- Wear, oxidation, or internal contact degradation → Increased insertion loss and intermittent connectivity

SYSTEM STATE:

- Degraded signal transmission across patch panel with unstable physical layer performance

CAUSAL RELATIONSHIPS:

[Signal Degradation] → [CRC/FCS Errors]
[Signal Degradation] → [Intermittent Link Flapping]
[Signal Degradation] → [Packet Loss (0.5–5%)]
[Signal Degradation] → [Latency / Jitter Increase]
[Signal Degradation] → [Application Timeouts]
[Signal Degradation] → [Increased MTTR]
[Signal Degradation] → [Monitoring Blind Spots]

IMPACT GROUPING:

• Network Stability:

- CRC/FCS errors

- Intermittent interface flapping

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Application timeouts

• Operational Impact:

- Increased MTTR

- Physical layer inspection and re-termination required

- Patch panel replacement or maintenance

• Security Implications:

- Monitoring gaps due to intermittent packet loss

- Reduced visibility in flow-based monitoring systems

    - Distance Exceedance

MECHANISM:

- Link distance beyond supported limit → Signal attenuation and low SNR causing high error rate under load

SYSTEM STATE:

- Marginal link with stable idle behavior but degradation under traffic conditions

CAUSAL RELATIONSHIPS:

[Low Signal-to-Noise Ratio] → [CRC/FCS Errors]
[Low Signal-to-Noise Ratio] → [Interface Flapping (under load)]
[Low Signal-to-Noise Ratio] → [Packet Loss (1–10%)]
[Low Signal-to-Noise Ratio] → [TCP Retransmissions]
[Low Signal-to-Noise Ratio] → [Throughput Degradation]
[Low Signal-to-Noise Ratio] → [Increased MTTR]
[Low Signal-to-Noise Ratio] → [Monitoring Blind Spots]

IMPACT GROUPING:

• Network Stability:

- CRC/FCS errors increase

- Load-dependent interface instability

• Service Disruption:

- Packet loss

- TCP retransmissions

- Reduced throughput / transfer failures

• Operational Impact:

- Increased MTTR

- Need for link redesign or signal amplification

- Diagnostic effort using optical tools

• Security Implications:

- Monitoring gaps during high error conditions

- Reduced accuracy of traffic visibility

  - Configuration Issues

    - Speed / Duplex Mismatch

MECHANISM:

- Auto-negotiation failure or mismatched manual configuration → One side operates in half-duplex while the other is full-duplex

SYSTEM STATE:

- Asymmetric link operation causing collisions, retransmissions, and degraded throughput

CAUSAL RELATIONSHIPS:

[Duplex Mismatch] → [Late Collisions]
[Duplex Mismatch] → [CRC/FCS Errors]
[Duplex Mismatch] → [Interface Flapping (error threshold)]
[Duplex Mismatch] → [Packet Loss (1–10%)]
[Duplex Mismatch] → [TCP Retransmissions]
[Duplex Mismatch] → [Severe Latency / Jitter]
[Duplex Mismatch] → [Increased MTTR]
[Duplex Mismatch] → [Authentication Timeouts]

IMPACT GROUPING:

• Network Stability:

- Late collisions (half-duplex side)

- CRC/FCS errors

- Interface resets due to error thresholds

• Service Disruption:

- Packet loss

- TCP retransmissions

- High latency / jitter (VoIP unusable)

• Operational Impact:

- Increased MTTR

- Troubleshooting complexity (hidden mismatch)

- Cross-team coordination (network/NIC/vendor)

• Security Implications:

- Authentication service disruptions (RADIUS/TACACS+)

- Reduced reliability of management plane access

    - STP Misconfiguration

MECHANISM:

- Missing PortFast or improper BPDU handling → Ports enter blocking/listening states or get err-disabled causing connectivity delays or link interruptions

SYSTEM STATE:

- Unstable Layer 2 topology with delayed forwarding or unintended port shutdowns

CAUSAL RELATIONSHIPS:

[STP Misbehavior] → [Interface State Delay / Flap]
[STP Misbehavior] → [MAC Table Flush]
[STP Misbehavior] → [STP Topology Change Notifications]
[STP Misbehavior] → [Packet Loss (during convergence)]
[STP Misbehavior] → [DHCP / Authentication Failures]
[STP Misbehavior] → [Increased MTTR]
[STP Misbehavior] → [Potential L2 Loop / Broadcast Storm]

IMPACT GROUPING:

• Network Stability:

- STP recalculation events

- MAC table flushing

- Interface transition delays

• Service Disruption:

- DHCP timeouts

- Authentication failures (802.1X)

- Packet loss during convergence

• Operational Impact:

- Increased MTTR

- Service desk ticket volume

- Configuration validation effort

• Security Implications:

- Potential Layer 2 loop exposure

- Compliance issues (STP hardening not enforced)

    - MTU Inconsistency

MECHANISM:

- MTU mismatch across network path → Larger packets dropped or fragmented inconsistently while smaller control packets pass

SYSTEM STATE:

- Partial connectivity where control plane traffic succeeds but data plane traffic fails for specific packet sizes

CAUSAL RELATIONSHIPS:

[MTU Mismatch] → [Packet Drops (Large Frames)]
[MTU Mismatch] → [TCP Retransmissions]
[MTU Mismatch] → [Application Timeouts]
[MTU Mismatch] → [Throughput Degradation]
[MTU Mismatch] → [Routing Protocol Instability (BGP Flaps)]
[MTU Mismatch] → [Increased MTTR]
[MTU Mismatch] → [Monitoring Blind Spots]

IMPACT GROUPING:

• Network Stability:

- Routing instability (BGP session flaps)

- Inconsistent forwarding behavior

• Service Disruption:

- Application timeouts

- File transfer failures

- Reduced throughput

• Operational Impact:

- Increased MTTR

- Complex troubleshooting (path MTU discovery issues)

- Configuration validation effort

• Security Implications:

- Compliance issues (MTU policy mismatch)

- Reduced visibility for large payload traffic

    - Port Security Violations

MECHANISM:

- Port security policy violation (MAC limit / sticky MAC mismatch) → Interface transitions to err-disable or restrict state

SYSTEM STATE:

- Access port blocked or intermittently disabled due to unauthorized or excess MAC address detection

CAUSAL RELATIONSHIPS:

[Security Violation] → [Interface Err-disable / Shutdown]
[Security Violation] → [Interface Flapping (auto-recovery)]
[Security Violation] → [MAC Address Drop]
[Security Violation] → [Packet Loss (10–100%)]
[Security Violation] → [Authentication Failures]
[Security Violation] → [Increased MTTR]
[Security Violation] → [Monitoring Blind Spots]

IMPACT GROUPING:

• Network Stability:

- Interface shutdown / err-disable

- MAC learning disruption

- Intermittent port recovery cycles

• Service Disruption:

- Complete loss of connectivity (affected port)

- Authentication failures

- Application access disruption

• Operational Impact:

- Increased MTTR

- Manual intervention required (port reset / reconfiguration)

- Security policy validation effort

• Security Implications:

- Intended protection mechanism triggered

- Potential visibility gaps during port disable state

    - VLAN Configuration Errors

MECHANISM:

- VLAN mismatch or incorrect tagging/untagging → Frames dropped or misrouted due to VLAN inconsistency

SYSTEM STATE:

- Logical segmentation inconsistency causing partial or complete loss of Layer 2/Layer 3 connectivity

CAUSAL RELATIONSHIPS:

[VLAN Mismatch] → [Frame Drops]
[VLAN Mismatch] → [MAC Table Inconsistency]
[VLAN Mismatch] → [Interface Flapping (logical)]
[VLAN Mismatch] → [Packet Loss (10–100%)]
[VLAN Mismatch] → [ARP Resolution Failure]
[VLAN Mismatch] → [Application Timeouts]
[VLAN Mismatch] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- MAC table inconsistencies

- Logical interface instability

- ARP cache issues

• Service Disruption:

- Packet loss

- Application timeouts

- Loss of connectivity across VLANs

• Operational Impact:

- Increased MTTR

- Configuration audit and correction effort

- Cross-device validation required

• Security Implications:

- VLAN leakage or segmentation failure

- Compliance violations (improper network isolation)

    - EtherChannel Misconfiguration

MECHANISM:

- LACP / EtherChannel parameter mismatch (system ID, key, mode, or member consistency) → Member links suspended or inconsistently bundled

SYSTEM STATE:

- Unstable aggregated link with intermittent member participation and inconsistent traffic distribution

CAUSAL RELATIONSHIPS:

[Channel Mismatch] → [Member Link Suspension]
[Channel Mismatch] → [Interface Flapping (Member Ports)]
[Channel Mismatch] → [MAC Flapping]
[Channel Mismatch] → [Packet Loss (1–10%)]
[Channel Mismatch] → [Throughput Degradation]
[Channel Mismatch] → [TCP Session Drops]
[Channel Mismatch] → [Increased MTTR]
[Channel Mismatch] → [Monitoring Blind Spots]

IMPACT GROUPING:

• Network Stability:

- Member ports added/removed dynamically

- MAC address flapping

- LACP state re-evaluation

• Service Disruption:

- Packet loss

- Inconsistent throughput

- TCP session interruptions

• Operational Impact:

- Increased MTTR

- Cross-device configuration validation required

- Troubleshooting complexity

• Security Implications:

- Traffic path unpredictability

- Reduced visibility in monitoring tools due to path changes

    - ACL / Filtering Rules

MECHANISM:

- Incorrect or overly restrictive ACL / filtering rules → Legitimate control or data traffic is blocked or rate-limited

SYSTEM STATE:

- Selective packet drops where link remains UP but critical traffic (control/data plane) is intermittently denied

CAUSAL RELATIONSHIPS:

[ACL Misconfiguration] → [Packet Drops (Selective)]
[ACL Misconfiguration] → [Routing Protocol Instability]
[ACL Misconfiguration] → [Neighbor Adjacency Flapping]
[ACL Misconfiguration] → [Application Timeouts]
[ACL Misconfiguration] → [TCP Retransmissions]
[ACL Misconfiguration] → [Increased MTTR]
[ACL Misconfiguration] → [Monitoring Blind Spots]

IMPACT GROUPING:

• Network Stability:

- Routing protocol flaps (OSPF/BGP keepalive drops)

- Neighbor adjacency instability

• Service Disruption:

- Application timeouts

- TCP retransmissions

- Partial or complete reachability loss

• Operational Impact:

- Increased MTTR

- Complex rule validation and troubleshooting

- Cross-team coordination (security/network)

• Security Implications:

- Over-blocking of legitimate traffic

- Potential blind spots in monitoring if telemetry is filtered

    - Routing Protocol Tuning Errors

MECHANISM:

- Incorrect routing protocol timers / thresholds (hello, dead, hold, BFD) → Premature neighbor timeout or delayed convergence

SYSTEM STATE:

- Unstable routing adjacencies with sensitivity to transient delays or packet loss

CAUSAL RELATIONSHIPS:

[Timer Misconfiguration] → [Neighbor Adjacency Flapping]
[Timer Misconfiguration] → [Routing Table Instability]
[Timer Misconfiguration] → [Frequent Convergence Events]
[Timer Misconfiguration] → [Packet Loss (during reconvergence)]
[Timer Misconfiguration] → [TCP Session Drops]
[Timer Misconfiguration] → [Increased MTTR]
[Timer Misconfiguration] → [Monitoring Blind Spots]

IMPACT GROUPING:

• Network Stability:

- Routing adjacency flaps (OSPF/BGP/BFD)

- Frequent route recalculations

• Service Disruption:

- Packet loss during convergence

- TCP session interruptions

- Intermittent connectivity

• Operational Impact:

- Increased MTTR

- Fine-tuning and validation effort

- Troubleshooting complexity (timing-related issues)

• Security Implications:

- Monitoring gaps during routing instability

- Reduced reliability of control-plane visibility

    - Interface Description Mismatch

MECHANISM:

- Incorrect or inconsistent interface descriptions → Operational misidentification leading to misconfiguration or incorrect troubleshooting actions

SYSTEM STATE:

- Logical misalignment between documented topology and actual connectivity causing configuration errors and unintended changes

CAUSAL RELATIONSHIPS:

[Misidentified Interface] → [Incorrect Configuration Applied]
[Misidentified Interface] → [Interface Shutdown / Flap]
[Misidentified Interface] → [Traffic Misrouting]
[Misidentified Interface] → [Packet Loss (Variable)]
[Misidentified Interface] → [Service Disruption]
[Misidentified Interface] → [Increased MTTR]
[Misidentified Interface] → [Operational Errors Propagation]

IMPACT GROUPING:

• Network Stability:

- Unintended interface changes

- Configuration inconsistencies

- Logical topology mismatch

• Service Disruption:

- Packet loss

- Misrouted or dropped traffic

- Intermittent connectivity issues

• Operational Impact:

- Increased MTTR

- Human error during troubleshooting

- Documentation correction effort

• Security Implications:

- Misapplied security policies

- Potential exposure due to incorrect interface targeting

    - CDP / LLDP Disabled

MECHANISM:

- CDP / LLDP disabled → Loss of neighbor discovery information leading to reduced topology visibility and delayed fault identification

SYSTEM STATE:

- Limited network visibility with missing neighbor relationships impacting troubleshooting and automation

CAUSAL RELATIONSHIPS:

[Loss of Topology Visibility] → [Delayed Fault Detection]
[Loss of Topology Visibility] → [Incorrect Troubleshooting Actions]
[Loss of Topology Visibility] → [Interface Misconfiguration]
[Loss of Topology Visibility] → [Traffic Misrouting]
[Loss of Topology Visibility] → [Service Disruption]
[Loss of Topology Visibility] → [Increased MTTR]
[Loss of Topology Visibility] → [Monitoring Blind Spots]

IMPACT GROUPING:

• Network Stability:

- Misidentified neighbor relationships

- Increased configuration inconsistencies

• Service Disruption:

- Misrouted traffic

- Intermittent connectivity due to incorrect fixes

• Operational Impact:

- Increased MTTR

- Higher dependency on manual verification

- Reduced automation effectiveness

• Security Implications:

- Reduced network visibility

- Potential gaps in topology-based security enforcement

  - Hardware/Software Issues

    - Switch ASIC Errors / Buffer Exhaustion

MECHANISM:

- ASIC buffer exhaustion (microbursts) or hardware processing errors → Packet drops and control-plane starvation leading to interface instability

SYSTEM STATE:

- Data plane congestion with control-plane packet loss causing intermittent protocol failures despite physical link being UP

CAUSAL RELATIONSHIPS:

[Buffer Exhaustion] → [Packet Drops (Burst 10–100%)]
[Buffer Exhaustion] → [Control Plane Packet Loss]
[Buffer Exhaustion] → [Interface Watchdog Reset]
[Buffer Exhaustion] → [Routing Protocol Flapping]
[Buffer Exhaustion] → [MAC Table Instability]
[Buffer Exhaustion] → [Increased MTTR]
[Buffer Exhaustion] → [Monitoring Blind Spots]

IMPACT GROUPING:

• Network Stability:

- Interface watchdog resets

- Routing protocol instability (OSPF/BGP/LDP)

- MAC table inconsistencies

• Service Disruption:

- Burst packet loss

- API / application failures

- Voice/video degradation

• Operational Impact:

- Increased MTTR

- Vendor escalation required

- Hardware-level diagnostics effort

• Security Implications:

- Packet drops before inspection

- Reduced effectiveness of intrusion detection systems

    - Firmware Bugs / NX-OS Defects

MECHANISM:

- Software defect (memory leak / protocol bug) → Process crash or line card reset causing interface instability

SYSTEM STATE:

- Control or data plane instability triggered by specific conditions or uptime thresholds leading to periodic link disruptions

CAUSAL RELATIONSHIPS:

[Software Defect] → [Process Crash / Restart]
[Software Defect] → [Interface Flapping]
[Software Defect] → [Module / Line Card Reset]
[Software Defect] → [Routing Protocol Reset]
[Software Defect] → [Packet Loss (Variable)]
[Software Defect] → [Increased MTTR]
[Software Defect] → [Monitoring Blind Spots]

IMPACT GROUPING:

• Network Stability:

- Interface flapping

- Routing protocol resets

- VPC / consistency issues

• Service Disruption:

- Intermittent or widespread connectivity loss

- Application interruptions

• Operational Impact:

- Increased MTTR

- Vendor support engagement

- Patch / upgrade planning effort

• Security Implications:

- Potential exploitability of known bugs

- Reduced visibility during process crashes

    - Thermal Overheating / Fan Failure

MECHANISM:

- Elevated temperature or fan failure → Hardware components enter thermal protection or become unstable causing interface shutdowns or flapping

SYSTEM STATE:

- Temperature-induced hardware instability leading to progressive degradation or sudden device shutdown

CAUSAL RELATIONSHIPS:

[Thermal Stress] → [Interface Flapping]
[Thermal Stress] → [ASIC / Hardware Errors]
[Thermal Stress] → [Device / Module Shutdown]
[Thermal Stress] → [Packet Loss (10–100%)]
[Thermal Stress] → [Routing / FHRP Failover]
[Thermal Stress] → [Increased MTTR]
[Thermal Stress] → [Monitoring Blind Spots]

IMPACT GROUPING:

• Network Stability:

- Interface instability

- Hardware errors

- Routing and FHRP transitions

• Service Disruption:

- Packet loss

- Partial or complete outage

- Service degradation under heat conditions

• Operational Impact:

- Increased MTTR

- Emergency cooling / shutdown procedures

- Hardware replacement or maintenance

• Security Implications:

- Monitoring gaps during shutdown events

- Reduced visibility during thermal-triggered failures

    - Memory Leak / CPU Exhaustion

MECHANISM:

- Memory leak or sustained high CPU utilization → Control plane starvation causing dropped protocol packets and delayed processing

SYSTEM STATE:

- Overloaded control plane with degraded responsiveness while physical interfaces remain UP

CAUSAL RELATIONSHIPS:

[Control Plane Exhaustion] → [Routing Protocol Flapping]
[Control Plane Exhaustion] → [BFD / Keepalive Drops]
[Control Plane Exhaustion] → [Interface Flapping (logical)]
[Control Plane Exhaustion] → [Packet Loss (Control/Data Plane)]
[Control Plane Exhaustion] → [Authentication / DHCP Failures]
[Control Plane Exhaustion] → [Increased MTTR]
[Control Plane Exhaustion] → [SIEM Log Gaps]

IMPACT GROUPING:

• Network Stability:

- Routing adjacency instability (OSPF/BGP/BFD)

- Delayed control-plane processing

• Service Disruption:

- Intermittent reachability

- Authentication and DHCP failures

- Packet loss impacting applications

• Operational Impact:

- Increased MTTR

- Resource-intensive troubleshooting

- Need for tuning / software fixes

• Security Implications:

- Log drops and monitoring gaps

- Increased susceptibility to control-plane attacks (DoS)

    - Backplane Congestion

MECHANISM:

- Internal backplane bandwidth saturation → Packet queuing and drops within switching fabric causing inconsistent forwarding

SYSTEM STATE:

- Internal switching congestion where ingress traffic exceeds fabric capacity leading to transient forwarding instability

CAUSAL RELATIONSHIPS:

[Backplane Congestion] → [Packet Drops (Burst 5–100%)]
[Backplane Congestion] → [Queue Build-up / Buffer Pressure]
[Backplane Congestion] → [Interface Flapping (threshold-triggered)]
[Backplane Congestion] → [Latency / Jitter Increase]
[Backplane Congestion] → [TCP Retransmissions]
[Backplane Congestion] → [Increased MTTR]
[Backplane Congestion] → [Monitoring Blind Spots]

IMPACT GROUPING:

• Network Stability:

- Internal fabric congestion

- Interface instability under load

- Queue and buffer pressure conditions

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Throughput degradation

• Operational Impact:

- Increased MTTR

- Capacity planning and redesign effort

- Hardware performance analysis required

• Security Implications:

- Packet drops before inspection

- Reduced visibility during congestion events

    - Faulty Line Cards / Modules

MECHANISM:

- Hardware defect in line card / module → Unstable port behavior or internal processing errors causing intermittent failures

SYSTEM STATE:

- Degraded module functionality leading to inconsistent interface operation and unpredictable packet handling

CAUSAL RELATIONSHIPS:

[Module Instability] → [Interface Flapping]
[Module Instability] → [CRC/FCS Errors]
[Module Instability] → [Packet Drops (Variable)]
[Module Instability] → [Routing / Protocol Instability]
[Module Instability] → [Service Interruption]
[Module Instability] → [Increased MTTR]
[Module Instability] → [Monitoring Blind Spots]

IMPACT GROUPING:

• Network Stability:

- Interface instability

- Hardware-induced errors

- Protocol inconsistencies

• Service Disruption:

- Packet loss

- Intermittent or partial outages

- Application disruptions

• Operational Impact:

- Increased MTTR

- Hardware replacement (RMA)

- Vendor diagnostics and escalation

• Security Implications:

- Reduced visibility due to unstable forwarding

- Potential gaps in traffic inspection

    - Power Supply Unit Failure

MECHANISM:

- Power Supply Unit failure → Sudden loss or instability of power to device or specific modules causing immediate shutdown or reset

SYSTEM STATE:

- Loss of power delivery leading to complete or partial device outage with all associated interfaces going down

CAUSAL RELATIONSHIPS:

[Power Loss] → [Device Shutdown / Reboot]
[Power Loss] → [Interface Down Events]
[Power Loss] → [Routing Protocol Reset]
[Power Loss] → [Packet Loss (100% during outage)]
[Power Loss] → [Service Outage]
[Power Loss] → [Increased MTTR]
[Power Loss] → [Monitoring Blind Spots]

IMPACT GROUPING:

• Network Stability:

- Complete interface down state

- Routing adjacency loss

- Network-wide convergence events

• Service Disruption:

- Full service outage

- Loss of connectivity

- Application and service failures

• Operational Impact:

- Increased MTTR

- Emergency hardware replacement (PSU)

- Incident escalation and recovery procedures

• Security Implications:

- Monitoring blind spots during outage

- Temporary loss of security enforcement controls

    - NVRAM / Flash Corruption

MECHANISM:

- Corruption of NVRAM / Flash storage → Configuration loss or failure to read system files causing unstable boot or runtime behavior

SYSTEM STATE:

- Inconsistent device state with potential reload loops, missing configuration, or partial feature failure

CAUSAL RELATIONSHIPS:

[Storage Corruption] → [Device Reload / Boot Failure]
[Storage Corruption] → [Configuration Loss / Mismatch]
[Storage Corruption] → [Interface Flapping]
[Storage Corruption] → [Routing / Protocol Instability]
[Storage Corruption] → [Packet Loss (Variable)]
[Storage Corruption] → [Increased MTTR]
[Storage Corruption] → [Monitoring Blind Spots]

IMPACT GROUPING:

• Network Stability:

- Device reload loops

- Interface instability

- Protocol inconsistencies

• Service Disruption:

- Partial or complete connectivity loss

- Application disruptions

- Intermittent service availability

• Operational Impact:

- Increased MTTR

- Configuration restoration effort

- Firmware reinstallation or recovery procedures

• Security Implications:

- Loss of security configurations

- Reduced visibility during unstable state

    - FPGA Programming Errors

MECHANISM:

- Incorrect or failed FPGA programming → Hardware logic operates unpredictably causing processing errors and interface instability

SYSTEM STATE:

- Faulty hardware behavior at silicon level leading to inconsistent packet processing and intermittent link failures

CAUSAL RELATIONSHIPS:

[FPGA Logic Error] → [Interface Flapping]
[FPGA Logic Error] → [Packet Processing Errors]
[FPGA Logic Error] → [CRC/FCS Errors]
[FPGA Logic Error] → [Packet Loss (Variable)]
[FPGA Logic Error] → [Routing / Protocol Instability]
[FPGA Logic Error] → [Increased MTTR]
[FPGA Logic Error] → [Monitoring Blind Spots]

IMPACT GROUPING:

• Network Stability:

- Interface instability

- Hardware-level processing errors

- Protocol inconsistencies

• Service Disruption:

- Packet loss

- Intermittent connectivity

- Application disruptions

• Operational Impact:

- Increased MTTR

- Firmware reprogramming or hardware replacement

- Vendor escalation

• Security Implications:

- Unpredictable packet handling

- Reduced reliability of traffic inspection

    - MAC Address Table Overflow

MECHANISM:

- MAC address table capacity exceeded → Switch floods unknown unicast traffic and may trigger protective mechanisms

SYSTEM STATE:

- Loss of deterministic forwarding with excessive flooding and potential interface instability

CAUSAL RELATIONSHIPS:

[MAC Table Overflow] → [Unknown Unicast Flooding]
[MAC Table Overflow] → [Broadcast-like Traffic Amplification]
[MAC Table Overflow] → [Interface Instability / Flapping]
[MAC Table Overflow] → [Packet Loss (Variable)]
[MAC Table Overflow] → [Network Congestion]
[MAC Table Overflow] → [Increased MTTR]
[MAC Table Overflow] → [Monitoring Blind Spots]

IMPACT GROUPING:

• Network Stability:

- MAC table instability

- Excessive flooding across VLAN

- Interface performance degradation

• Service Disruption:

- Packet loss

- Network congestion

- Application performance issues

• Operational Impact:

- Increased MTTR

- Root cause analysis effort (possible attack vs misconfig)

- Network redesign or segmentation required

• Security Implications:

- Potential MAC flooding attack scenario

- Increased attack surface due to traffic flooding

  - External Factors

    - Upstream Provider Issues

MECHANISM:

- Upstream network instability or outage → Loss or degradation of external connectivity causing route withdrawal or interface down events

SYSTEM STATE:

- Dependency failure where local network remains stable but upstream path becomes unavailable or unstable

CAUSAL RELATIONSHIPS:

[Upstream Instability] → [BGP Session Flap / Reset]
[Upstream Instability] → [Route Withdrawal]
[Upstream Instability] → [FHRP Failover]
[Upstream Instability] → [Packet Loss (10–100%)]
[Upstream Instability] → [DNS / External Service Failure]
[Upstream Instability] → [Increased MTTR]
[Upstream Instability] → [Monitoring Blind Spots]

IMPACT GROUPING:

• Network Stability:

- BGP session instability

- Routing changes / failover events

- MPLS / WAN path disruptions

• Service Disruption:

- Loss of internet / WAN connectivity

- DNS resolution failures

- Application outages (external dependencies)

• Operational Impact:

- Increased MTTR

- Vendor coordination required

- Escalation and incident management overhead

• Security Implications:

- Monitoring blind spots for external traffic

- Exposure risk during failover or degraded routing

    - Power Grid Instability

MECHANISM:

- External power grid instability (outages, voltage fluctuations) → Inconsistent power supply to network infrastructure causing device resets or shutdowns

SYSTEM STATE:

- Unreliable external power source leading to intermittent or complete loss of device operation

CAUSAL RELATIONSHIPS:

[Grid Instability] → [Device Shutdown / Reboot]
[Grid Instability] → [Interface Down Events]
[Grid Instability] → [Routing Protocol Reset]
[Grid Instability] → [Packet Loss (10–100%)]
[Grid Instability] → [Service Outage]
[Grid Instability] → [Increased MTTR]
[Grid Instability] → [Monitoring Blind Spots]

IMPACT GROUPING:

• Network Stability:

- Multiple device outages

- Routing adjacency loss

- Network convergence events

• Service Disruption:

- Complete or partial service outage

- Loss of connectivity

- Application downtime

• Operational Impact:

- Increased MTTR

- Dependency on backup power systems (UPS/generator)

- Incident escalation and recovery procedures

• Security Implications:

- Monitoring blind spots during outages

- Temporary loss of security enforcement mechanisms

    - Weather Events / Natural Disasters

MECHANISM:

- Extreme weather or natural disaster impact → Physical infrastructure damage or environmental stress causing link failure or instability

SYSTEM STATE:

- External environmental disruption leading to partial or complete network infrastructure failure

CAUSAL RELATIONSHIPS:

[Infrastructure Damage] → [Fiber / Link Failure]
[Infrastructure Damage] → [Device Shutdown / Failure]
[Infrastructure Damage] → [Interface Down Events]
[Infrastructure Damage] → [Routing Protocol Reset]
[Infrastructure Damage] → [Packet Loss (50–100%)]
[Infrastructure Damage] → [Service Outage]
[Infrastructure Damage] → [Increased MTTR]
[Infrastructure Damage] → [Monitoring Blind Spots]

IMPACT GROUPING:

• Network Stability:

- Complete link or node failure

- Routing adjacency loss

- Network-wide convergence events

• Service Disruption:

- Extended service outage

- Loss of connectivity

- Application downtime

• Operational Impact:

- Increased MTTR

- Disaster recovery procedures activation

- Infrastructure repair and coordination effort

• Security Implications:

- Monitoring blind spots during outage

- Reduced enforcement of security controls

- Increased risk exposure during recovery

    - Construction / Fiber Cuts

MECHANISM:

- Physical fiber damage due to construction activity → Immediate loss of optical signal causing link down events

SYSTEM STATE:

- Complete break in transmission medium leading to total loss of connectivity on affected path

CAUSAL RELATIONSHIPS:

[Fiber Cut] → [Interface Down Events]
[Fiber Cut] → [BGP Session Reset]
[Fiber Cut] → [Route Withdrawal]
[Fiber Cut] → [Packet Loss (100%)]
[Fiber Cut] → [Service Outage]
[Fiber Cut] → [Increased MTTR]
[Fiber Cut] → [Monitoring Blind Spots]

IMPACT GROUPING:

• Network Stability:

- Interface down state

- Routing adjacency loss

- Network convergence events

• Service Disruption:

- Complete connectivity loss

- Application outages

- WAN / external service disruption

• Operational Impact:

- Increased MTTR

- Coordination with provider / field teams

- Physical repair and restoration effort

• Security Implications:

- Monitoring blind spots during outage

- Reduced visibility of external traffic

    - DDoS / Security Attacks

MECHANISM:

- Malicious traffic flood (control plane or data plane saturation) → Resource exhaustion causing packet drops and protocol instability

SYSTEM STATE:

- Overloaded network elements with degraded control/data plane performance despite physical link being UP

CAUSAL RELATIONSHIPS:

[Traffic Flood] → [Control Plane Overload]
[Traffic Flood] → [BGP/OSPF Flapping]
[Traffic Flood] → [Packet Drops (10–100%)]
[Traffic Flood] → [Interface Congestion]
[Traffic Flood] → [Application Timeouts]
[Traffic Flood] → [Increased MTTR]
[Traffic Flood] → [SIEM Log Gaps]

IMPACT GROUPING:

• Network Stability:

- Routing protocol instability

- Interface congestion

- MAC / control plane anomalies

• Service Disruption:

- Packet loss

- Application timeouts

- Authentication / service failures

• Operational Impact:

- Increased MTTR

- Incident response and mitigation effort

- Vendor / ISP coordination (scrubbing, filtering)

• Security Implications:

- Active attack scenario

- SIEM log gaps due to overload

- Potential intrusion detection bypass

    - Third-Party Maintenance Windows

MECHANISM:

- Scheduled maintenance by third-party provider → Planned service interruption or session reset causing temporary link instability

SYSTEM STATE:

- External dependency undergoing controlled changes leading to predictable but disruptive connectivity behavior

CAUSAL RELATIONSHIPS:

[Planned Maintenance] → [Interface Down / Reset]
[Planned Maintenance] → [BGP Session Flap]
[Planned Maintenance] → [Route Withdrawal]
[Planned Maintenance] → [Packet Loss (10–100% during window)]
[Planned Maintenance] → [Service Interruption]
[Planned Maintenance] → [Increased MTTR]
[Planned Maintenance] → [Monitoring Blind Spots]

IMPACT GROUPING:

• Network Stability:

- Routing changes during maintenance

- Temporary adjacency loss

- Controlled convergence events

• Service Disruption:

- Temporary connectivity loss

- Application interruptions

- API / replication failures

• Operational Impact:

- Increased MTTR (if misattributed)

- Change management coordination (CAB)

- Dependency tracking effort

• Security Implications:

- Planned monitoring blind spots

- Reduced visibility during maintenance window

    - Regulatory / Compliance Changes

MECHANISM:

- Enforcement of new regulatory or compliance policies → Configuration changes (ACLs, encryption, segmentation) causing unintended traffic blocking or instability

SYSTEM STATE:

- Policy-driven network behavior changes leading to partial reachability and control/data plane inconsistencies

CAUSAL RELATIONSHIPS:

[Policy Change] → [Traffic Filtering / Blocking]
[Policy Change] → [Routing / Access Instability]
[Policy Change] → [Authentication Failures]
[Policy Change] → [Packet Loss (Variable)]
[Policy Change] → [Application Timeouts]
[Policy Change] → [Increased MTTR]
[Policy Change] → [Monitoring Blind Spots]

IMPACT GROUPING:

• Network Stability:

- Routing or access inconsistencies

- Policy-induced control plane impact

• Service Disruption:

- Application timeouts

- Authentication failures

- Partial connectivity loss

• Operational Impact:

- Increased MTTR

- Policy validation and rollback effort

- Cross-team coordination (security/compliance/network)

• Security Implications:

- Over-restrictive enforcement

- Compliance-driven changes impacting visibility

- Potential gaps during transition phase

    - DNS Provider Outages

MECHANISM:

- DNS provider outage or instability → Failure in name resolution despite underlying network connectivity being intact

SYSTEM STATE:

- Control plane dependency failure where services are reachable by IP but inaccessible via domain names

CAUSAL RELATIONSHIPS:

[DNS Failure] → [Name Resolution Failure]
[DNS Failure] → [Application Timeouts]
[DNS Failure] → [Service Reachability Issues]
[DNS Failure] → [Perceived Network Flapping]
[DNS Failure] → [API / External Service Failures]
[DNS Failure] → [Increased MTTR]
[DNS Failure] → [Monitoring Blind Spots]

IMPACT GROUPING:

• Network Stability:

- No direct link instability

- Perceived instability due to resolution failures

• Service Disruption:

- Application access failure

- API and external dependency failure

- Intermittent service availability (cached vs non-cached)

• Operational Impact:

- Increased MTTR

- Misdiagnosis as network issue

- Dependency troubleshooting effort

• Security Implications:

- Potential DNS spoofing suspicion

- Reduced visibility in domain-based monitoring

    - Cloud Provider Region Failures

MECHANISM:

- Cloud region outage or service degradation → Loss of connectivity to hosted resources or control plane endpoints

SYSTEM STATE:

- External dependency failure where regional infrastructure becomes unavailable or unstable

CAUSAL RELATIONSHIPS:

[Region Outage] → [Service Endpoint Unreachable]
[Region Outage] → [BGP / Route Changes]
[Region Outage] → [Packet Loss (10–100%)]
[Region Outage] → [Application Failures]
[Region Outage] → [Failover Triggered]
[Region Outage] → [Increased MTTR]
[Region Outage] → [Monitoring Blind Spots]

IMPACT GROUPING:

• Network Stability:

- Route changes / failover events

- External path instability

• Service Disruption:

- Application outages

- API failures

- Loss of access to cloud-hosted services

• Operational Impact:

- Increased MTTR

- Failover and recovery management

- Vendor coordination (cloud provider)

• Security Implications:

- Monitoring gaps in affected region

- Reduced visibility during failover conditions

    - Cable Theft / Vandalism

MECHANISM:

- Physical cable damage or removal due to theft / vandalism → Complete loss of transmission medium causing immediate link failure

SYSTEM STATE:

- Sudden and unplanned physical disconnection leading to total loss of connectivity on affected links

CAUSAL RELATIONSHIPS:

[Cable Damage] → [Interface Down Events]
[Cable Damage] → [Routing Protocol Reset]
[Cable Damage] → [Route Withdrawal]
[Cable Damage] → [Packet Loss (100%)]
[Cable Damage] → [Service Outage]
[Cable Damage] → [Increased MTTR]
[Cable Damage] → [Monitoring Blind Spots]

IMPACT GROUPING:

• Network Stability:

- Immediate interface down state

- Routing adjacency loss

- Network convergence events

• Service Disruption:

- Complete connectivity loss

- Application outages

- WAN / external service disruption

• Operational Impact:

- Increased MTTR

- Physical inspection and repair effort

- Coordination with law enforcement / providers

• Security Implications:

- Monitoring blind spots during outage

- Physical security breach exposure

- Increased risk of repeated incidents

Event 2: Network Congestion

  - Traffic Volume / Capacity

    - Sudden Traffic Spike (Flash Crowd, Marketing Campaign)

MECHANISM:

- Sudden surge in legitimate traffic demand → Link utilization exceeds capacity causing queue buildup and packet drops

SYSTEM STATE:

- High bandwidth saturation with congested interfaces and increased queuing delays

CAUSAL RELATIONSHIPS:

[Traffic Surge] → [High Utilization (≥80–100%)]
[Traffic Surge] → [Queue Build-up]
[Traffic Surge] → [Packet Drops (0.5–20%)]
[Traffic Surge] → [Increased Latency / Jitter]
[Traffic Surge] → [TCP Retransmissions]
[Traffic Surge] → [Application Timeouts]
[Traffic Surge] → [Increased MTTR]
[Traffic Surge] → [Monitoring Blind Spots]

IMPACT GROUPING:

• Network Stability:

- Interface congestion

- Queue saturation

- Control plane delays under load

• Service Disruption:

- Application slowdowns

- Packet loss

- Increased latency / jitter

- API / web request failures

• Operational Impact:

- Increased MTTR

- Traffic engineering / rate limiting required

- Capacity planning adjustments

• Security Implications:

- Traffic resembles DoS patterns (false positives)

- Monitoring blind spots due to overload

- Reduced effectiveness of anomaly detection

    - Distributed Denial-of-Service (DDoS) Attack

MECHANISM:

- Malicious distributed traffic flood → Bandwidth and device resources overwhelmed causing sustained congestion and packet drops

SYSTEM STATE:

- Saturated links and overloaded control/data plane with inability to process legitimate traffic

CAUSAL RELATIONSHIPS:

[DDoS Traffic Flood] → [High Utilization (90–100%)]
[DDoS Traffic Flood] → [Queue Saturation]
[DDoS Traffic Flood] → [Packet Drops (10–100%)]
[DDoS Traffic Flood] → [Increased Latency / Jitter]
[DDoS Traffic Flood] → [TCP Retransmissions]
[DDoS Traffic Flood] → [Application / Service Failure]
[DDoS Traffic Flood] → [Control Plane Overload]
[DDoS Traffic Flood] → [Increased MTTR]
[DDoS Traffic Flood] → [SIEM Log Gaps]

IMPACT GROUPING:

• Network Stability:

- Interface congestion

- Control plane instability

- Routing protocol degradation under load

• Service Disruption:

- Severe packet loss

- Application outages

- Authentication / DNS failures

• Operational Impact:

- Increased MTTR

- Incident response and mitigation effort (scrubbing, rate limiting)

- Vendor / ISP coordination

• Security Implications:

- Active attack condition (DoS)

- SIEM log loss due to overload

- Intrusion detection bypass

- Increased attack surface exposure

    - Misconfigured Load Balancer Causing Asymmetric Flows

MECHANISM:

- Load balancer misconfiguration → Uneven traffic distribution causing asymmetric flows and hotspot links

SYSTEM STATE:

- Imbalanced path utilization where some links are congested while others remain underutilized

CAUSAL RELATIONSHIPS:

[Asymmetric Traffic Flow] → [Hotspot Link Congestion]
[Asymmetric Traffic Flow] → [High Utilization (≥90% on specific links)]
[Asymmetric Traffic Flow] → [Queue Build-up]
[Asymmetric Traffic Flow] → [Packet Drops (1–20%)]
[Asymmetric Traffic Flow] → [Increased Latency / Jitter]
[Asymmetric Traffic Flow] → [TCP Retransmissions]
[Asymmetric Traffic Flow] → [Application Timeouts]
[Asymmetric Traffic Flow] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Uneven link utilization

- Congestion on specific paths

- Suboptimal routing behavior

• Service Disruption:

- Packet loss on congested links

- Increased latency / jitter

- Intermittent application failures

• Operational Impact:

- Increased MTTR

- Load balancer tuning required

- Traffic engineering and path optimization effort

• Security Implications:

- Reduced visibility due to asymmetric paths

- Monitoring inconsistencies across links

    - East-West Traffic Growth (Microservices, BackupsCloud Burst / Data Replication Events

MECHANISM:

- Increased East-West traffic (microservices, backups, replication) → Internal network links saturate due to high intra-data-center traffic volume

SYSTEM STATE:

- Heavy internal traffic flows causing congestion within aggregation/core layers despite stable external connectivity

CAUSAL RELATIONSHIPS:

[East-West Traffic Growth] → [High Utilization (≥80–100%)]
[East-West Traffic Growth] → [Queue Build-up]
[East-West Traffic Growth] → [Packet Drops (1–25%)]
[East-West Traffic Growth] → [Increased Latency / Jitter]
[East-West Traffic Growth] → [TCP Retransmissions]
[East-West Traffic Growth] → [Application / Service Slowdown]
[East-West Traffic Growth] → [Replication / Backup Failures]
[East-West Traffic Growth] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Core/aggregation congestion

- Fabric-level resource pressure

- Potential control plane delays under load

• Service Disruption:

- Application slowdowns (microservices latency)

- Backup / replication timeouts

- Increased latency / jitter

• Operational Impact:

- Increased MTTR

- Capacity planning adjustments

- Traffic engineering (segmentation / scheduling)

• Security Implications:

- Monitoring blind spots due to high internal traffic volume

- Reduced visibility for east-west traffic inspection

    - Software Update / Patch Distribution

MECHANISM:

- Large-scale software update / patch distribution → Simultaneous bulk data transfer saturating network links

SYSTEM STATE:

- Burst-driven bandwidth consumption causing congestion across access, aggregation, and WAN links

CAUSAL RELATIONSHIPS:

[Bulk Update Traffic] → [High Utilization (≥80–100%)]
[Bulk Update Traffic] → [Queue Build-up]
[Bulk Update Traffic] → [Packet Drops (1–20%)]
[Bulk Update Traffic] → [Increased Latency / Jitter]
[Bulk Update Traffic] → [TCP Retransmissions]
[Bulk Update Traffic] → [Application Slowdown]
[Bulk Update Traffic] → [Update Failures / Retries]
[Bulk Update Traffic] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Link congestion across multiple segments

- Queue saturation

- Potential control-plane delays under heavy load

• Service Disruption:

- Application performance degradation

- Slow or failed update distribution

- Increased latency / jitter

• Operational Impact:

- Increased MTTR

- Need for staged rollout / scheduling

- Bandwidth management and throttling required

• Security Implications:

- Monitoring blind spots due to traffic spikes

- Reduced effectiveness of anomaly detection during bulk transfers

    - Video Conferencing Surge (All-Hands Meeting)

MECHANISM:

- Simultaneous real-time video conferencing sessions → High volume of latency-sensitive UDP traffic saturating available bandwidth

SYSTEM STATE:

- Bandwidth contention with prioritization challenges between real-time traffic and regular application flows

CAUSAL RELATIONSHIPS:

[Video Traffic Surge] → [High Utilization (≥80–100%)]
[Video Traffic Surge] → [Queue Build-up]
[Video Traffic Surge] → [Packet Drops (1–15%)]
[Video Traffic Surge] → [Increased Latency / Jitter]
[Video Traffic Surge] → [UDP Packet Loss]
[Video Traffic Surge] → [Voice/Video Quality Degradation]
[Video Traffic Surge] → [Application Slowdown]
[Video Traffic Surge] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Interface congestion

- QoS contention between traffic classes

- Increased buffer pressure

• Service Disruption:

- Video freezing / pixelation

- Voice jitter / call drops

- Degraded performance of other applications

• Operational Impact:

- Increased MTTR

- QoS policy tuning required

- Bandwidth planning for peak events

• Security Implications:

- Monitoring blind spots due to high-volume UDP traffic

- Reduced visibility in encrypted video streams

    - IoT Device Firmware Updates

MECHANISM:

- Mass IoT firmware update rollout → Simultaneous device downloads generating bursty distributed traffic across access and aggregation layers

SYSTEM STATE:

- High fan-out traffic pattern with numerous endpoints consuming bandwidth concurrently, leading to widespread congestion

CAUSAL RELATIONSHIPS:

[IoT Update Traffic] → [High Utilization (≥80–100%)]
[IoT Update Traffic] → [Queue Build-up]
[IoT Update Traffic] → [Packet Drops (1–20%)]
[IoT Update Traffic] → [Increased Latency / Jitter]
[IoT Update Traffic] → [TCP Retransmissions]
[IoT Update Traffic] → [Device Update Failures / Retries]
[IoT Update Traffic] → [Application Slowdown]
[IoT Update Traffic] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Access/aggregation layer congestion

- Burst traffic from large number of endpoints

- Increased buffer pressure

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Degraded performance of enterprise applications

• Operational Impact:

- Increased MTTR

- Need for staged rollout / device grouping

- Bandwidth management and scheduling

• Security Implications:

- Monitoring blind spots due to distributed traffic spikes

- Reduced visibility across numerous IoT endpoints

    - BGP Route Leak / Hijack

MECHANISM:

- BGP route leak or hijack → Incorrect route advertisement causing traffic redirection and unintended path congestion

SYSTEM STATE:

- Suboptimal or malicious routing paths leading to traffic concentration on unintended links

CAUSAL RELATIONSHIPS:

[Route Leak / Hijack] → [Suboptimal Routing Paths]
[Route Leak / Hijack] → [Traffic Concentration on Specific Links]
[Route Leak / Hijack] → [High Utilization (≥80–100%)]
[Route Leak / Hijack] → [Queue Build-up]
[Route Leak / Hijack] → [Packet Drops (1–30%)]
[Route Leak / Hijack] → [Increased Latency / Jitter]
[Route Leak / Hijack] → [TCP Retransmissions]
[Route Leak / Hijack] → [Service Reachability Issues]
[Route Leak / Hijack] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Routing instability

- Unexpected path changes

- Congestion on unintended links

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Partial or complete service inaccessibility

• Operational Impact:

- Increased MTTR

- Routing investigation and correction effort

- Coordination with upstream providers

• Security Implications:

- Potential traffic interception (hijack scenario)

- Monitoring blind spots due to path deviation

- Increased risk of data exposure

    - Multicast Flooding / Unknown Unicast Flood

MECHANISM:

- Multicast flooding or unknown unicast flooding → Frames replicated across multiple ports due to lack of proper forwarding entries or control mechanisms

SYSTEM STATE:

- Excessive Layer 2 traffic propagation causing unnecessary bandwidth consumption across switches

CAUSAL RELATIONSHIPS:

[Flooded Traffic] → [High Utilization (≥80–100%)]
[Flooded Traffic] → [Interface Congestion]
[Flooded Traffic] → [Queue Build-up]
[Flooded Traffic] → [Packet Drops (1–25%)]
[Flooded Traffic] → [Increased Latency / Jitter]
[Flooded Traffic] → [MAC Table Instability]
[Flooded Traffic] → [Application Slowdown]
[Flooded Traffic] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- MAC table instability

- Broadcast/multicast amplification

- Interface congestion across VLAN

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Degraded application performance

• Operational Impact:

- Increased MTTR

- Troubleshooting effort (identifying flood source)

- Need for IGMP snooping / storm control configuration

• Security Implications:

- Potential L2 attack scenario (flooding attack)

- Monitoring blind spots due to excessive traffic

- Increased attack surface within broadcast domain

  - Configuration Issues

    - QoS Misconfiguration (Missing or Incorrect Policies)

MECHANISM:

- Missing or incorrect QoS policies → Traffic not properly classified or prioritized causing critical flows to compete equally with bulk traffic

SYSTEM STATE:

- Uncontrolled traffic scheduling leading to inefficient bandwidth utilization and starvation of latency-sensitive applications

CAUSAL RELATIONSHIPS:

[QoS Misconfiguration] → [Improper Traffic Classification]
[QoS Misconfiguration] → [Queue Imbalance]
[QoS Misconfiguration] → [Queue Build-up]
[QoS Misconfiguration] → [Packet Drops (1–20%)]
[QoS Misconfiguration] → [Increased Latency / Jitter]
[QoS Misconfiguration] → [Voice/Video Degradation]
[QoS Misconfiguration] → [Application Slowdown]
[QoS Misconfiguration] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Queue imbalance across traffic classes

- Inefficient bandwidth allocation

- Buffer pressure conditions

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Voice/video quality degradation

- Application performance issues

• Operational Impact:

- Increased MTTR

- QoS policy redesign and tuning

- Traffic classification validation effort

• Security Implications:

- Monitoring inaccuracies due to misclassified traffic

- Reduced effectiveness of traffic-based anomaly detection

    - Buffer Size / Queue Depth Mismanagement

MECHANISM:

- Improper buffer allocation or queue depth configuration → Buffers either overflow (too small) or induce excessive latency (too large)

SYSTEM STATE:

- Inefficient queue management causing packet drops under burst traffic or increased delay due to bufferbloat

CAUSAL RELATIONSHIPS:

[Buffer Mismanagement] → [Queue Overflow]
[Buffer Mismanagement] → [Excessive Queueing Delay]
[Buffer Mismanagement] → [Packet Drops (1–25%)]
[Buffer Mismanagement] → [Increased Latency / Jitter]
[Buffer Mismanagement] → [TCP Retransmissions]
[Buffer Mismanagement] → [Throughput Degradation]
[Buffer Mismanagement] → [Application Slowdown]
[Buffer Mismanagement] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Queue instability under burst traffic

- Buffer pressure / overflow conditions

- Inefficient traffic scheduling

• Service Disruption:

- Packet loss

- Increased latency / jitter (bufferbloat)

- Reduced throughput

• Operational Impact:

- Increased MTTR

- Queue tuning and buffer reallocation effort

- Performance analysis required

• Security Implications:

- Monitoring blind spots during burst drops

- Reduced visibility due to delayed or dropped packets

    - ACL / Firewall Rule Misorder

MECHANISM:

- Incorrect ACL / firewall rule order → Higher-priority rules unintentionally block or rate-limit legitimate traffic before intended matches

SYSTEM STATE:

- Misapplied filtering logic causing selective traffic drops and uneven load distribution across network paths

CAUSAL RELATIONSHIPS:

[Rule Misordering] → [Legitimate Traffic Drops]
[Rule Misordering] → [Traffic Imbalance]
[Rule Misordering] → [Queue Build-up]
[Rule Misordering] → [Packet Drops (1–20%)]
[Rule Misordering] → [Increased Latency / Jitter]
[Rule Misordering] → [TCP Retransmissions]
[Rule Misordering] → [Application Timeouts]
[Rule Misordering] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Traffic flow inconsistencies

- Uneven load across paths

- Policy-induced congestion

• Service Disruption:

- Application timeouts

- Packet loss

- Degraded service performance

• Operational Impact:

- Increased MTTR

- Rule validation and reordering effort

- Cross-team coordination (network/security)

• Security Implications:

- Over-blocking or unintended exposure

- Reduced monitoring accuracy due to incorrect filtering

    - STP Topology Change Causing Broadcast Storm

MECHANISM:

- STP topology change or loop condition → Uncontrolled broadcast and unknown unicast flooding saturating Layer 2 domains

SYSTEM STATE:

- Loop-induced traffic amplification causing exponential increase in frame replication across switches

CAUSAL RELATIONSHIPS:

[Broadcast Storm] → [High Utilization (≥90–100%)]
[Broadcast Storm] → [Interface Congestion]
[Broadcast Storm] → [Queue Build-up]
[Broadcast Storm] → [Packet Drops (10–100%)]
[Broadcast Storm] → [MAC Table Instability]
[Broadcast Storm] → [CPU Spike (Control Plane)]
[Broadcast Storm] → [Application Failure]
[Broadcast Storm] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- MAC table instability

- Control plane overload

- Widespread L2 congestion

• Service Disruption:

- Severe packet loss

- Network-wide slowdown or outage

- Application failures

• Operational Impact:

- Increased MTTR

- Emergency loop isolation required

- STP configuration correction effort

• Security Implications:

- Potential L2 attack scenario (loop or storm)

- Monitoring blind spots due to excessive traffic

- Intrusion detection system overload

    - Routing Protocol Metric Misconfiguration

MECHANISM:

- Incorrect routing metrics (cost, weight, preference) → Traffic routed through suboptimal paths causing uneven load distribution

SYSTEM STATE:

- Inefficient path selection leading to congestion on certain links while alternate paths remain underutilized

CAUSAL RELATIONSHIPS:

[Metric Misconfiguration] → [Suboptimal Path Selection]
[Metric Misconfiguration] → [Traffic Concentration]
[Metric Misconfiguration] → [High Utilization (≥80–100%)]
[Metric Misconfiguration] → [Queue Build-up]
[Metric Misconfiguration] → [Packet Drops (1–20%)]
[Metric Misconfiguration] → [Increased Latency / Jitter]
[Metric Misconfiguration] → [TCP Retransmissions]
[Metric Misconfiguration] → [Application Slowdown]
[Metric Misconfiguration] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Uneven traffic distribution

- Congestion on preferred paths

- Underutilization of alternate paths

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Degraded application performance

• Operational Impact:

- Increased MTTR

- Routing policy correction effort

- Traffic engineering adjustments

• Security Implications:

- Monitoring inconsistencies across paths

- Reduced visibility due to asymmetric routing

    - ECMP Hashing Imbalance

MECHANISM:

- Uneven ECMP hash distribution → Flows mapped disproportionately to specific paths causing localized congestion

SYSTEM STATE:

- Parallel paths available but traffic not evenly balanced, leading to hotspot links while others remain underutilized

CAUSAL RELATIONSHIPS:

[Hash Imbalance] → [Uneven Traffic Distribution]
[Hash Imbalance] → [Hotspot Link Congestion]
[Hash Imbalance] → [High Utilization (≥80–100% on specific links)]
[Hash Imbalance] → [Queue Build-up]
[Hash Imbalance] → [Packet Drops (1–20%)]
[Hash Imbalance] → [Increased Latency / Jitter]
[Hash Imbalance] → [TCP Retransmissions]
[Hash Imbalance] → [Application Slowdown]
[Hash Imbalance] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Path imbalance across ECMP links

- Congestion on selected paths

- Underutilization of alternate paths

• Service Disruption:

- Packet loss on congested links

- Increased latency / jitter

- Intermittent application performance issues

• Operational Impact:

- Increased MTTR

- Hash algorithm tuning / adjustment

- Traffic engineering effort

• Security Implications:

- Monitoring inconsistencies due to path imbalance

- Reduced visibility across uneven traffic distribution

    - VLAN Misassignment

MECHANISM:

- Incorrect VLAN assignment → Traffic forwarded to unintended broadcast domains causing congestion and misrouted flows

SYSTEM STATE:

- Logical segmentation failure leading to traffic mixing across VLANs and inefficient forwarding behavior

CAUSAL RELATIONSHIPS:

[VLAN Misassignment] → [Traffic Misrouting]
[VLAN Misassignment] → [Unnecessary Broadcast / Flooding]
[VLAN Misassignment] → [High Utilization (≥80–100%)]
[VLAN Misassignment] → [Queue Build-up]
[VLAN Misassignment] → [Packet Drops (1–20%)]
[VLAN Misassignment] → [Increased Latency / Jitter]
[VLAN Misassignment] → [Application Timeouts]
[VLAN Misassignment] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Broadcast domain expansion

- MAC table inconsistencies

- Increased L2 traffic load

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Application connectivity issues

• Operational Impact:

- Increased MTTR

- VLAN configuration audit and correction

- Cross-device validation effort

• Security Implications:

- VLAN leakage / segmentation failure

- Increased attack surface within broadcast domain

- Monitoring inconsistencies

    - MTU Mismatch Leading to Fragmentation

MECHANISM:

- MTU mismatch across path → Packet fragmentation or drops causing inefficient transmission and increased processing overhead

SYSTEM STATE:

- Fragmented or dropped packets leading to reduced effective throughput and increased latency

CAUSAL RELATIONSHIPS:

[MTU Mismatch] → [Packet Fragmentation]
[MTU Mismatch] → [Packet Drops (DF set)]
[MTU Mismatch] → [High Utilization (inefficient bandwidth use)]
[MTU Mismatch] → [Queue Build-up]
[MTU Mismatch] → [Increased Latency / Jitter]
[MTU Mismatch] → [TCP Retransmissions]
[MTU Mismatch] → [Throughput Degradation]
[MTU Mismatch] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Inefficient packet forwarding

- Increased processing overhead on devices

- Potential routing inconsistencies

• Service Disruption:

- Application timeouts

- Reduced throughput

- File transfer inefficiencies

• Operational Impact:

- Increased MTTR

- Path MTU troubleshooting effort

- Configuration validation across devices

• Security Implications:

- Monitoring blind spots for fragmented traffic

- Reduced visibility in packet inspection systems

    - Port Channel / LAG Load-Sharing Imbalance

MECHANISM:

- Uneven load sharing across Port Channel / LAG members → Traffic concentrated on specific links due to hashing limitations or configuration mismatch

SYSTEM STATE:

- Aggregated links available but ineffective utilization leading to hotspot congestion on selected member interfaces

CAUSAL RELATIONSHIPS:

[LAG Imbalance] → [Uneven Traffic Distribution]
[LAG Imbalance] → [Hotspot Link Congestion]
[LAG Imbalance] → [High Utilization (≥80–100% on specific members)]
[LAG Imbalance] → [Queue Build-up]
[LAG Imbalance] → [Packet Drops (1–20%)]
[LAG Imbalance] → [Increased Latency / Jitter]
[LAG Imbalance] → [TCP Retransmissions]
[LAG Imbalance] → [Application Slowdown]
[LAG Imbalance] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Member link imbalance

- Congestion on specific interfaces

- Underutilization of other links

• Service Disruption:

- Packet loss on congested links

- Increased latency / jitter

- Intermittent application performance degradation

• Operational Impact:

- Increased MTTR

- Hashing algorithm tuning required

- Traffic engineering adjustments

• Security Implications:

- Monitoring inconsistencies across member links

- Reduced visibility due to uneven traffic distribution

    - Congestion Avoidance (WRED) Thresholds Too High

MECHANISM:

- WRED thresholds set too high → Packet drops delayed until buffers are nearly full causing bufferbloat and sudden tail drops

SYSTEM STATE:

- Excessive queue occupancy leading to high latency followed by burst packet loss under sustained congestion

CAUSAL RELATIONSHIPS:

[High WRED Thresholds] → [Excessive Queue Build-up]
[High WRED Thresholds] → [Bufferbloat]
[High WRED Thresholds] → [Increased Latency / Jitter]
[High WRED Thresholds] → [Sudden Packet Drops (5–30%)]
[High WRED Thresholds] → [TCP Global Synchronization]
[High WRED Thresholds] → [Throughput Degradation]
[High WRED Thresholds] → [Application Slowdown]
[High WRED Thresholds] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Inefficient congestion control behavior

- Large queue occupancy

- Sudden drop patterns affecting stability

• Service Disruption:

- High latency / jitter (bufferbloat)

- Burst packet loss

- Reduced application responsiveness

• Operational Impact:

- Increased MTTR

- QoS/WRED tuning required

- Performance analysis and validation effort

• Security Implications:

- Monitoring blind spots due to delayed drops

- Reduced effectiveness of traffic anomaly detection

  - Hardware / Software Limitations

    - Oversubscribed Switching Fabric

MECHANISM:

- Switching fabric oversubscription → Aggregate ingress traffic exceeds internal fabric capacity causing contention and packet drops

SYSTEM STATE:

- Internal switching bottleneck where multiple high-speed ports compete for limited backplane bandwidth

CAUSAL RELATIONSHIPS:

[Fabric Oversubscription] → [Internal Contention]
[Fabric Oversubscription] → [Queue Build-up]
[Fabric Oversubscription] → [Packet Drops (1–30%)]
[Fabric Oversubscription] → [Increased Latency / Jitter]
[Fabric Oversubscription] → [Throughput Degradation]
[Fabric Oversubscription] → [TCP Retransmissions]
[Fabric Oversubscription] → [Application Slowdown]
[Fabric Oversubscription] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Internal fabric congestion

- Contention across multiple ports

- Reduced forwarding efficiency

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Degraded application performance

• Operational Impact:

- Increased MTTR

- Capacity planning and hardware upgrade requirements

- Traffic engineering effort

• Security Implications:

- Monitoring blind spots due to internal drops

- Reduced visibility of traffic during congestion

    - Insufficient Port Buffers

MECHANISM:

- Insufficient port buffer capacity → Inability to absorb traffic bursts leading to immediate packet drops during microbursts

SYSTEM STATE:

- Burst-sensitive interfaces where short-duration traffic spikes cause frequent packet loss despite moderate average utilization

CAUSAL RELATIONSHIPS:

[Low Buffer Capacity] → [Microburst Packet Drops]
[Low Buffer Capacity] → [Queue Overflow]
[Low Buffer Capacity] → [Packet Drops (1–40% during bursts)]
[Low Buffer Capacity] → [Increased Latency / Jitter]
[Low Buffer Capacity] → [TCP Retransmissions]
[Low Buffer Capacity] → [Throughput Degradation]
[Low Buffer Capacity] → [Application Slowdown]
[Low Buffer Capacity] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Burst-induced instability

- Queue overflow conditions

- Sensitivity to traffic spikes

• Service Disruption:

- Packet loss during bursts

- Increased latency / jitter

- Degraded application performance

• Operational Impact:

- Increased MTTR

- Hardware limitation analysis

- Need for traffic shaping or buffer tuning

• Security Implications:

- Monitoring blind spots during microbursts

- Reduced visibility due to transient packet loss

    - Underpowered CPU for Control Plane

MECHANISM:

- Insufficient CPU capacity for control plane processing → Inability to handle routing updates, keepalives, and management traffic under load

SYSTEM STATE:

- Overloaded control plane with delayed or dropped protocol packets while data plane traffic continues

CAUSAL RELATIONSHIPS:

[CPU Overload] → [Control Plane Packet Drops]
[CPU Overload] → [Routing Protocol Instability]
[CPU Overload] → [BGP/OSPF Neighbor Flaps]
[CPU Overload] → [Increased Latency (Control Processing)]
[CPU Overload] → [Packet Loss (Control/Data Plane)]
[CPU Overload] → [Application Timeouts]
[CPU Overload] → [Increased MTTR]
[CPU Overload] → [SIEM Log Gaps]

IMPACT GROUPING:

• Network Stability:

- Routing adjacency instability

- Delayed control-plane processing

- Protocol convergence issues

• Service Disruption:

- Intermittent connectivity

- Application timeouts

- Authentication / DHCP delays

• Operational Impact:

- Increased MTTR

- CPU tuning or hardware upgrade required

- Control-plane policing adjustments

• Security Implications:

- Log loss and monitoring gaps

- Increased susceptibility to control-plane attacks (DoS)

    - ASIC TCAM / FIB Exhaustion

MECHANISM:

- TCAM / FIB table exhaustion → New entries cannot be installed causing fallback to software forwarding or packet drops

SYSTEM STATE:

- Forwarding table saturation leading to degraded hardware switching efficiency and increased control-plane involvement

CAUSAL RELATIONSHIPS:

[Table Exhaustion] → [Route / ACL Entry Installation Failure]
[Table Exhaustion] → [Software Forwarding (CPU Path)]
[Table Exhaustion] → [High CPU Utilization]
[Table Exhaustion] → [Packet Drops (1–30%)]
[Table Exhaustion] → [Increased Latency / Jitter]
[Table Exhaustion] → [Routing Instability]
[Table Exhaustion] → [Application Slowdown]
[Table Exhaustion] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Forwarding inconsistencies

- Increased CPU dependency

- Routing / policy instability

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Degraded application performance

• Operational Impact:

- Increased MTTR

- Hardware capacity upgrade or optimization required

- Policy / route scaling adjustments

• Security Implications:

- ACL enforcement limitations

- Monitoring inaccuracies due to forwarding inconsistencies

    - Line Card Backplane Congestion

MECHANISM:

- Line card backplane congestion → Internal bandwidth between line card and switching fabric becomes saturated causing forwarding delays and drops

SYSTEM STATE:

- Localized internal congestion where traffic from specific line cards exceeds available backplane capacity

CAUSAL RELATIONSHIPS:

[Backplane Congestion] → [Internal Queue Build-up]
[Backplane Congestion] → [Packet Drops (1–30%)]
[Backplane Congestion] → [High Utilization (Line Card Level)]
[Backplane Congestion] → [Increased Latency / Jitter]
[Backplane Congestion] → [Throughput Degradation]
[Backplane Congestion] → [TCP Retransmissions]
[Backplane Congestion] → [Application Slowdown]
[Backplane Congestion] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Line card-level congestion

- Uneven traffic distribution across modules

- Internal fabric contention

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Degraded application performance

• Operational Impact:

- Increased MTTR

- Hardware capacity analysis required

- Traffic redistribution or redesign

• Security Implications:

- Monitoring blind spots within congested modules

- Reduced visibility of traffic affected by internal drops

    - Firmware Bug in Queue Management

MECHANISM:

- Firmware defect in queue management → Incorrect scheduling, buffering, or drop behavior causing inefficient congestion handling

SYSTEM STATE:

- Unpredictable queue behavior with improper packet scheduling, leading to either excessive drops or high latency

CAUSAL RELATIONSHIPS:

[Queue Management Bug] → [Incorrect Packet Scheduling]
[Queue Management Bug] → [Queue Build-up / Starvation]
[Queue Management Bug] → [Packet Drops (1–30%)]
[Queue Management Bug] → [Increased Latency / Jitter]
[Queue Management Bug] → [TCP Retransmissions]
[Queue Management Bug] → [Throughput Degradation]
[Queue Management Bug] → [Application Slowdown]
[Queue Management Bug] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Unstable queue behavior

- Inefficient congestion handling

- Traffic class starvation

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Degraded application performance

• Operational Impact:

- Increased MTTR

- Firmware upgrade / patch required

- Vendor support engagement

• Security Implications:

- Monitoring inaccuracies due to unpredictable drops

- Reduced effectiveness of traffic analysis

    - Memory Leak in Routing Process

MECHANISM:

- Memory leak in routing process → Gradual resource exhaustion leading to degraded control-plane performance and instability

SYSTEM STATE:

- Increasing memory utilization causing delayed routing updates, process restarts, or control-plane degradation

CAUSAL RELATIONSHIPS:

[Memory Leak] → [High Memory Utilization]
[Memory Leak] → [Routing Process Slowdown]
[Memory Leak] → [Control Plane Instability]
[Memory Leak] → [Routing Update Delays]
[Memory Leak] → [Packet Loss (Control/Data Plane)]
[Memory Leak] → [Application Timeouts]
[Memory Leak] → [Increased MTTR]
[Memory Leak] → [SIEM Log Gaps]

IMPACT GROUPING:

• Network Stability:

- Routing instability

- Delayed convergence

- Control-plane degradation

• Service Disruption:

- Intermittent connectivity

- Application timeouts

- Packet loss

• Operational Impact:

- Increased MTTR

- Process restart or device reload required

- Firmware patch / bug fix effort

• Security Implications:

- Log loss and monitoring gaps

- Reduced visibility during degraded state

    - MAC Address Table Overflow

MECHANISM:

- MAC address table overflow → Switch resorts to flooding unknown unicast traffic causing excessive bandwidth consumption

SYSTEM STATE:

- Loss of efficient Layer 2 forwarding leading to broadcast-like traffic propagation across the network

CAUSAL RELATIONSHIPS:

[MAC Table Overflow] → [Unknown Unicast Flooding]
[MAC Table Overflow] → [High Utilization (≥80–100%)]
[MAC Table Overflow] → [Queue Build-up]
[MAC Table Overflow] → [Packet Drops (1–25%)]
[MAC Table Overflow] → [Increased Latency / Jitter]
[MAC Table Overflow] → [Network Congestion]
[MAC Table Overflow] → [Application Slowdown]
[MAC Table Overflow] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- MAC learning instability

- Excessive flooding across VLANs

- Increased Layer 2 traffic load

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Degraded application performance

• Operational Impact:

- Increased MTTR

- Root cause investigation (attack vs misconfig)

- Network segmentation or scaling effort

• Security Implications:

- Potential MAC flooding attack scenario

- Increased attack surface due to traffic flooding

- Monitoring inconsistencies

    - NetFlow / sFlow Sampling Overload

MECHANISM:

- Excessive NetFlow / sFlow sampling load → CPU and memory overhead increases due to high rate of flow processing and export

SYSTEM STATE:

- Monitoring overhead impacting control/data plane performance under high traffic conditions

CAUSAL RELATIONSHIPS:

[Sampling Overload] → [High CPU Utilization]
[Sampling Overload] → [Control Plane Processing Delay]
[Sampling Overload] → [Packet Drops (Control/Data Plane)]
[Sampling Overload] → [Increased Latency / Jitter]
[Sampling Overload] → [Flow Export Delay / Loss]
[Sampling Overload] → [Application Slowdown]
[Sampling Overload] → [Increased MTTR]
[Sampling Overload] → [SIEM Log Gaps]

IMPACT GROUPING:

• Network Stability:

- Control plane performance degradation

- Delayed packet processing

- Increased CPU pressure

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Degraded application performance

• Operational Impact:

- Increased MTTR

- Monitoring configuration tuning required

- Trade-off between visibility and performance

• Security Implications:

- Loss of flow visibility

- SIEM data gaps

- Reduced effectiveness of anomaly detection

  - External Factors

    - Upstream ISP Peering Congestion

MECHANISM:

- Congestion at upstream ISP peering points → Limited bandwidth at interconnects causing queuing and packet drops for external traffic

SYSTEM STATE:

- External path saturation where local network is healthy but upstream links are congested

CAUSAL RELATIONSHIPS:

[Peering Congestion] → [High Utilization (≥80–100% at peering links)]
[Peering Congestion] → [Queue Build-up]
[Peering Congestion] → [Packet Drops (1–30%)]
[Peering Congestion] → [Increased Latency / Jitter]
[Peering Congestion] → [TCP Retransmissions]
[Peering Congestion] → [Application Slowdown]
[Peering Congestion] → [Service Reachability Issues]
[Peering Congestion] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- External path congestion

- Suboptimal routing behavior

- Increased dependency on alternate paths

• Service Disruption:

- Packet loss to external services

- Increased latency / jitter

- Degraded performance for internet-facing applications

• Operational Impact:

- Increased MTTR

- ISP coordination required

- Traffic rerouting or peering adjustments

• Security Implications:

- Monitoring blind spots for external traffic

- Reduced visibility into upstream congestion events

    - CDN Provider Outage / Throttling

MECHANISM:

- CDN outage or throttling → Traffic redirected to fewer or distant nodes causing bandwidth concentration and increased latency

SYSTEM STATE:

- Reduced content distribution efficiency leading to hotspot links and longer delivery paths

CAUSAL RELATIONSHIPS:

[CDN Degradation] → [Traffic Concentration]
[CDN Degradation] → [High Utilization (≥80–100%)]
[CDN Degradation] → [Queue Build-up]
[CDN Degradation] → [Packet Drops (1–25%)]
[CDN Degradation] → [Increased Latency / Jitter]
[CDN Degradation] → [TCP Retransmissions]
[CDN Degradation] → [Application Slowdown]
[CDN Degradation] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Uneven traffic distribution

- External path congestion

- Increased dependency on fewer nodes

• Service Disruption:

- Slow content delivery

- Packet loss

- Increased latency / jitter

- Degraded user experience

• Operational Impact:

- Increased MTTR

- CDN provider coordination

- Traffic rerouting or failover strategy

• Security Implications:

- Monitoring blind spots due to traffic rerouting

- Reduced visibility into CDN-managed traffic

    - Cloud Provider Regional Saturation

MECHANISM:

- Cloud region resource saturation → Limited compute/network capacity causing throttling and congestion at provider edge

SYSTEM STATE:

- External dependency bottleneck where regional infrastructure cannot handle incoming/outgoing traffic demand

CAUSAL RELATIONSHIPS:

[Regional Saturation] → [Traffic Throttling]
[Regional Saturation] → [High Utilization (Provider Edge)]
[Regional Saturation] → [Queue Build-up]
[Regional Saturation] → [Packet Drops (1–30%)]
[Regional Saturation] → [Increased Latency / Jitter]
[Regional Saturation] → [TCP Retransmissions]
[Regional Saturation] → [Application Timeouts]
[Regional Saturation] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- External path congestion

- Reduced performance at cloud ingress/egress points

- Dependency-driven instability

• Service Disruption:

- Application slowdowns

- API latency / failures

- Packet loss and timeouts

• Operational Impact:

- Increased MTTR

- Cloud provider coordination

- Load redistribution / multi-region failover planning

• Security Implications:

- Monitoring blind spots within saturated region

- Reduced visibility into cloud-managed traffic flows

    - DNS Amplification Attack

MECHANISM:

- DNS amplification attack → Small spoofed queries trigger large responses causing traffic amplification and bandwidth saturation

SYSTEM STATE:

- Excessive inbound/outbound UDP traffic overwhelming network links and intermediate devices

CAUSAL RELATIONSHIPS:

[Amplified DNS Traffic] → [High Utilization (≥90–100%)]
[Amplified DNS Traffic] → [Queue Build-up]
[Amplified DNS Traffic] → [Packet Drops (10–100%)]
[Amplified DNS Traffic] → [Increased Latency / Jitter]
[Amplified DNS Traffic] → [UDP Packet Loss]
[Amplified DNS Traffic] → [Service Disruption]
[Amplified DNS Traffic] → [Control Plane Overload]
[Amplified DNS Traffic] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Interface saturation

- Control plane stress due to high packet rate

- Routing instability under load

• Service Disruption:

- Severe packet loss

- DNS resolution failures

- Application outages

• Operational Impact:

- Increased MTTR

- Incident response and mitigation (rate limiting, filtering)

- Coordination with ISP / scrubbing providers

• Security Implications:

- Active DDoS condition

- Intrusion detection overload

- Monitoring blind spots due to traffic volume

- Potential SIEM data gaps

    - Undersea Cable Outage

MECHANISM:

- Undersea cable outage → Loss of major international links causing traffic rerouting over limited alternate paths

SYSTEM STATE:

- Global path disruption leading to congestion on backup routes and increased latency for long-haul traffic

CAUSAL RELATIONSHIPS:

[Cable Outage] → [Traffic Rerouting]
[Cable Outage] → [Alternate Path Congestion]
[Cable Outage] → [High Utilization (≥80–100%)]
[Cable Outage] → [Queue Build-up]
[Cable Outage] → [Packet Drops (1–30%)]
[Cable Outage] → [Increased Latency / Jitter]
[Cable Outage] → [TCP Retransmissions]
[Cable Outage] → [Application Slowdown]
[Cable Outage] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- External path instability

- Congestion on backup routes

- Increased routing convergence events

• Service Disruption:

- Increased latency (long-haul paths)

- Packet loss

- Degraded application performance

• Operational Impact:

- Increased MTTR

- ISP coordination and traffic rerouting

- Capacity and resiliency planning adjustments

• Security Implications:

- Monitoring blind spots due to rerouted traffic

- Reduced visibility across alternate paths

    - Power Outage Affecting PoP

MECHANISM:

- Power outage at Point of Presence (PoP) → Sudden loss of edge infrastructure causing traffic rerouting to alternate nodes

SYSTEM STATE:

- Regional access point failure leading to load shift and congestion on remaining PoPs and transit links

CAUSAL RELATIONSHIPS:

[PoP Power Loss] → [Node Unavailability]
[PoP Power Loss] → [Traffic Rerouting]
[PoP Power Loss] → [Alternate Path Congestion]
[PoP Power Loss] → [High Utilization (≥80–100%)]
[PoP Power Loss] → [Queue Build-up]
[PoP Power Loss] → [Packet Drops (1–30%)]
[PoP Power Loss] → [Increased Latency / Jitter]
[PoP Power Loss] → [Application Slowdown]
[PoP Power Loss] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Regional node failure

- Routing convergence events

- Congestion on alternate paths

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Partial service degradation

• Operational Impact:

- Increased MTTR

- Incident escalation and provider coordination

- Failover and capacity planning adjustments

• Security Implications:

- Monitoring blind spots in affected region

- Reduced visibility during failover conditions

    - Natural Disaster Impacting Fiber Routes

MECHANISM:

- Natural disaster damaging multiple fiber routes → Simultaneous path failures forcing traffic onto limited backup links

SYSTEM STATE:

- Widespread physical infrastructure disruption causing severe congestion on remaining network paths

CAUSAL RELATIONSHIPS:

[Fiber Route Damage] → [Multiple Link Failures]
[Fiber Route Damage] → [Traffic Rerouting]
[Fiber Route Damage] → [Alternate Path Congestion]
[Fiber Route Damage] → [High Utilization (≥80–100%)]
[Fiber Route Damage] → [Queue Build-up]
[Fiber Route Damage] → [Packet Drops (5–50%)]
[Fiber Route Damage] → [Increased Latency / Jitter]
[Fiber Route Damage] → [Application Slowdown]
[Fiber Route Damage] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Multiple path failures

- Heavy congestion on backup routes

- Increased routing convergence activity

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Degraded application performance or outages

• Operational Impact:

- Increased MTTR

- Disaster recovery coordination

- Infrastructure repair and rerouting efforts

• Security Implications:

- Monitoring blind spots across affected regions

- Reduced visibility due to path rerouting

- Increased exposure during degraded network conditions

    - Third-Party Application API Rate Limiting

MECHANISM:

- Third-party API rate limiting → Excess requests throttled or delayed causing retries and amplified traffic patterns

SYSTEM STATE:

- Application-layer throttling leading to repeated request bursts and inefficient bandwidth utilization

CAUSAL RELATIONSHIPS:

[API Rate Limiting] → [Request Retries]
[API Rate Limiting] → [Traffic Amplification]
[API Rate Limiting] → [High Utilization (≥80–100%)]
[API Rate Limiting] → [Queue Build-up]
[API Rate Limiting] → [Packet Drops (1–20%)]
[API Rate Limiting] → [Increased Latency / Jitter]
[API Rate Limiting] → [Application Timeouts]
[API Rate Limiting] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Traffic amplification due to retries

- Increased load on network paths

- Inefficient bandwidth utilization

• Service Disruption:

- Application timeouts

- API failures

- Increased latency / jitter

• Operational Impact:

- Increased MTTR

- Dependency troubleshooting effort

- API usage optimization required

• Security Implications:

- Monitoring blind spots due to retry storms

- Potential misclassification as attack traffic

- Reduced visibility into third-party controlled flows

Event 3: ROUTING RECONVERGENCE

  - Network Topology Changes

    - Link / Node Failure (Physical Interface Down)

MECHANISM:

- Physical link or node failure → Traffic redirected to alternate paths increasing load on remaining network resources

SYSTEM STATE:

- Reduced network capacity with surviving links handling additional traffic leading to congestion

CAUSAL RELATIONSHIPS:

[Link/Node Failure] → [Traffic Rerouting]
[Link/Node Failure] → [Reduced Available Bandwidth]
[Link/Node Failure] → [High Utilization (≥80–100%)]
[Link/Node Failure] → [Queue Build-up]
[Link/Node Failure] → [Packet Drops (1–30%)]
[Link/Node Failure] → [Increased Latency / Jitter]
[Link/Node Failure] → [TCP Retransmissions]
[Link/Node Failure] → [Application Slowdown]
[Link/Node Failure] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Path failover events

- Congestion on remaining links

- Routing convergence activity

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Degraded application performance

• Operational Impact:

- Increased MTTR

- Fault isolation and recovery effort

- Capacity rebalancing

• Security Implications:

- Monitoring blind spots on failed path

- Reduced visibility during rerouting

    - Link Flap / BFD Session Drop

MECHANISM:

- Intermittent link flap or BFD session drop → Frequent path failover causing repeated traffic rerouting and instability

SYSTEM STATE:

- Fluctuating network paths with unstable forwarding behavior leading to inconsistent load distribution

CAUSAL RELATIONSHIPS:

[Link Flap / BFD Drop] → [Frequent Traffic Rerouting]
[Link Flap / BFD Drop] → [Path Instability]
[Link Flap / BFD Drop] → [High Utilization (≥80–100% on alternate paths)]
[Link Flap / BFD Drop] → [Queue Build-up]
[Link Flap / BFD Drop] → [Packet Drops (1–25%)]
[Link Flap / BFD Drop] → [Increased Latency / Jitter]
[Link Flap / BFD Drop] → [TCP Retransmissions]
[Link Flap / BFD Drop] → [Application Instability]
[Link Flap / BFD Drop] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Frequent routing convergence events

- Path oscillation

- Uneven traffic distribution

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Intermittent application failures

• Operational Impact:

- Increased MTTR

- Troubleshooting complexity (intermittent issue)

- Need for stabilization (timers / thresholds tuning)

• Security Implications:

- Monitoring blind spots during flaps

- Reduced visibility due to rapid path changes

    - Planned Maintenance (Shutdown, Reload)

MECHANISM:

- Planned maintenance (shutdown / reload) → Temporary removal of links or nodes causing traffic redistribution to remaining paths

SYSTEM STATE:

- Controlled reduction in network capacity with predictable load increase on alternate paths

CAUSAL RELATIONSHIPS:

[Maintenance Event] → [Link/Node Unavailability]
[Maintenance Event] → [Traffic Rerouting]
[Maintenance Event] → [Reduced Available Bandwidth]
[Maintenance Event] → [High Utilization (≥80–100%)]
[Maintenance Event] → [Queue Build-up]
[Maintenance Event] → [Packet Drops (1–20%)]
[Maintenance Event] → [Increased Latency / Jitter]
[Maintenance Event] → [Application Slowdown]
[Maintenance Event] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Planned path failover

- Temporary congestion on alternate links

- Predictable routing changes

• Service Disruption:

- Minor to moderate packet loss

- Increased latency / jitter

- Temporary application degradation

• Operational Impact:

- Increased MTTR (if misplanned)

- Change management coordination (CAB)

- Maintenance window planning and validation

• Security Implications:

- Planned monitoring blind spots

- Reduced visibility during maintenance window

    - Cable Cut / Fiber Break

MECHANISM:

- Physical cable cut or fiber break → Immediate loss of primary path forcing traffic onto alternate routes

SYSTEM STATE:

- Sudden reduction in network capacity with congestion on backup links handling redirected traffic

CAUSAL RELATIONSHIPS:

[Cable/Fiber Break] → [Primary Path Failure]
[Cable/Fiber Break] → [Traffic Rerouting]
[Cable/Fiber Break] → [Reduced Available Bandwidth]
[Cable/Fiber Break] → [High Utilization (≥80–100%)]
[Cable/Fiber Break] → [Queue Build-up]
[Cable/Fiber Break] → [Packet Drops (1–30%)]
[Cable/Fiber Break] → [Increased Latency / Jitter]
[Cable/Fiber Break] → [TCP Retransmissions]
[Cable/Fiber Break] → [Application Slowdown]
[Cable/Fiber Break] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Path failure and rerouting

- Congestion on alternate links

- Routing convergence events

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Degraded application performance

• Operational Impact:

- Increased MTTR

- Physical repair coordination

- Provider / field team involvement

• Security Implications:

- Monitoring blind spots on failed path

- Reduced visibility during rerouting

    - Power Failure (Switch/Router, PoP)

MECHANISM:

- Power failure at switch/router or PoP → Sudden loss of network node causing traffic redistribution to remaining infrastructure

SYSTEM STATE:

- Node outage leading to reduced network capacity and increased load on alternate paths

CAUSAL RELATIONSHIPS:

[Power Failure] → [Node Unavailability]
[Power Failure] → [Traffic Rerouting]
[Power Failure] → [Reduced Available Bandwidth]
[Power Failure] → [High Utilization (≥80–100%)]
[Power Failure] → [Queue Build-up]
[Power Failure] → [Packet Drops (1–30%)]
[Power Failure] → [Increased Latency / Jitter]
[Power Failure] → [Application Slowdown]
[Power Failure] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Node failure and rerouting

- Congestion on alternate paths

- Routing convergence events

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Partial or complete service degradation

• Operational Impact:

- Increased MTTR

- Emergency recovery procedures

- Power infrastructure validation

• Security Implications:

- Monitoring blind spots during outage

- Reduced visibility in affected region

    - Spanning-Tree Topology Change (Indirect)

MECHANISM:

- Spanning Tree topology change → Traffic paths recalculated causing temporary loops, flooding, or suboptimal forwarding

SYSTEM STATE:

- Layer 2 instability with transient path changes leading to uneven traffic distribution and temporary congestion

CAUSAL RELATIONSHIPS:

[STP Topology Change] → [Path Recalculation]
[STP Topology Change] → [Temporary Flooding]
[STP Topology Change] → [MAC Table Flush]
[STP Topology Change] → [High Utilization (≥80–100%)]
[STP Topology Change] → [Queue Build-up]
[STP Topology Change] → [Packet Drops (1–25%)]
[STP Topology Change] → [Increased Latency / Jitter]
[STP Topology Change] → [Application Instability]
[STP Topology Change] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- MAC table instability

- Temporary loops or flooding

- Path recalculation events

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Intermittent connectivity issues

• Operational Impact:

- Increased MTTR

- STP tuning and validation effort

- Troubleshooting complexity

• Security Implications:

- Monitoring blind spots during topology changes

- Increased exposure due to temporary flooding

    - MPLS LSP / TE Tunnel Flap

MECHANISM:

- MPLS LSP / Traffic Engineering tunnel flap → Frequent path re-signaling and rerouting causing traffic shifts and transient congestion

SYSTEM STATE:

- Unstable MPLS forwarding paths with oscillating tunnel availability leading to uneven load distribution

CAUSAL RELATIONSHIPS:

[LSP / TE Flap] → [Frequent Traffic Rerouting]
[LSP / TE Flap] → [Path Oscillation]
[LSP / TE Flap] → [High Utilization (≥80–100% on alternate paths)]
[LSP / TE Flap] → [Queue Build-up]
[LSP / TE Flap] → [Packet Drops (1–25%)]
[LSP / TE Flap] → [Increased Latency / Jitter]
[LSP / TE Flap] → [TCP Retransmissions]
[LSP / TE Flap] → [Application Instability]
[LSP / TE Flap] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- MPLS path instability

- Frequent rerouting events

- Uneven traffic distribution

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Intermittent application performance issues

• Operational Impact:

- Increased MTTR

- MPLS/TE tuning and troubleshooting

- Coordination across network domains

• Security Implications:

- Monitoring blind spots during tunnel flaps

- Reduced visibility due to path oscillation

    - VPC / VSS Split-Brain Event

MECHANISM:

- VPC / VSS split-brain condition → Dual active forwarding leading to duplicate traffic, loops, or inconsistent forwarding decisions

SYSTEM STATE:

- Control-plane inconsistency between paired devices causing parallel active paths and traffic duplication

CAUSAL RELATIONSHIPS:

[Split Brain Condition] → [Duplicate Traffic Forwarding]
[Split Brain Condition] → [Layer 2 Loop / Flooding]
[Split Brain Condition] → [High Utilization (≥90–100%)]
[Split Brain Condition] → [Queue Build-up]
[Split Brain Condition] → [Packet Drops (5–50%)]
[Split Brain Condition] → [MAC Table Instability]
[Split Brain Condition] → [Increased Latency / Jitter]
[Split Brain Condition] → [Application Failure]
[Split Brain Condition] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- MAC flapping and instability

- L2 loop conditions

- Control-plane inconsistency

• Service Disruption:

- Severe packet loss

- Network-wide instability

- Application outages

• Operational Impact:

- Increased MTTR

- Complex fault isolation (dual-active condition)

- Recovery and synchronization effort

• Security Implications:

- Monitoring blind spots during instability

- Increased attack surface due to uncontrolled forwarding

- Potential bypass of security controls

  - Protocol Configuration Issues

    - BGP: Maximum Prefix Limit Exceeded

MECHANISM:

- BGP maximum prefix limit exceeded → Session reset or route suppression causing sudden withdrawal of routes

SYSTEM STATE:

- Loss of routing information leading to traffic rerouting over limited alternate paths

CAUSAL RELATIONSHIPS:

[Prefix Limit Exceeded] → [BGP Session Reset / Route Suppression]
[Prefix Limit Exceeded] → [Route Withdrawal]
[Prefix Limit Exceeded] → [Traffic Rerouting]
[Prefix Limit Exceeded] → [Reduced Available Bandwidth]
[Prefix Limit Exceeded] → [High Utilization (≥80–100%)]
[Prefix Limit Exceeded] → [Queue Build-up]
[Prefix Limit Exceeded] → [Packet Drops (1–30%)]
[Prefix Limit Exceeded] → [Increased Latency / Jitter]
[Prefix Limit Exceeded] → [Application Slowdown]
[Prefix Limit Exceeded] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- BGP session instability

- Routing table changes

- Path convergence events

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Partial reachability issues

• Operational Impact:

- Increased MTTR

- BGP policy tuning required

- Coordination with upstream peers

• Security Implications:

- Monitoring blind spots due to route loss

- Potential misinterpretation as attack (route leak/hijack)

    - BGP: Peer Reset / Soft Reconfiguration

MECHANISM:

- BGP peer reset or soft reconfiguration → Temporary withdrawal and reinstallation of routes causing traffic shifts

SYSTEM STATE:

- Transient routing instability with recalculation of best paths and redistribution of traffic

CAUSAL RELATIONSHIPS:

[BGP Reset/Reconfig] → [Route Withdrawal / Reinstallation]
[BGP Reset/Reconfig] → [Traffic Rerouting]
[BGP Reset/Reconfig] → [Reduced Available Bandwidth]
[BGP Reset/Reconfig] → [High Utilization (≥80–100%)]
[BGP Reset/Reconfig] → [Queue Build-up]
[BGP Reset/Reconfig] → [Packet Drops (1–25%)]
[BGP Reset/Reconfig] → [Increased Latency / Jitter]
[BGP Reset/Reconfig] → [TCP Retransmissions]
[BGP Reset/Reconfig] → [Application Instability]
[BGP Reset/Reconfig] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Routing table recalculation

- Temporary path instability

- Convergence events

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Intermittent application performance

• Operational Impact:

- Increased MTTR

- Routing policy validation effort

- Coordination with peers/providers

• Security Implications:

- Monitoring blind spots during route changes

- Reduced visibility due to path shifts

    - OSPF: Hello / Dead Interval Mismatch

MECHANISM:

- OSPF hello/dead interval mismatch → Neighbor adjacency fails or flaps causing route instability

SYSTEM STATE:

- Inconsistent OSPF adjacency leading to intermittent route availability and traffic redistribution

CAUSAL RELATIONSHIPS:

[OSPF Interval Mismatch] → [Neighbor Adjacency Failure/Flap]
[OSPF Interval Mismatch] → [Route Withdrawal / Reinstallation]
[OSPF Interval Mismatch] → [Traffic Rerouting]
[OSPF Interval Mismatch] → [Reduced Available Bandwidth]
[OSPF Interval Mismatch] → [High Utilization (≥80–100%)]
[OSPF Interval Mismatch] → [Queue Build-up]
[OSPF Interval Mismatch] → [Packet Drops (1–25%)]
[OSPF Interval Mismatch] → [Increased Latency / Jitter]
[OSPF Interval Mismatch] → [Application Instability]
[OSPF Interval Mismatch] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- OSPF adjacency instability

- Frequent convergence events

- Routing inconsistencies

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Intermittent connectivity

• Operational Impact:

- Increased MTTR

- Configuration correction effort

- Protocol tuning and validation

• Security Implications:

- Monitoring blind spots during adjacency flaps

- Reduced visibility due to unstable routing

    - OSPF: LSA Flooding / Database Overload

MECHANISM:

- Excessive OSPF LSA flooding → Control-plane overload due to frequent database updates and SPF recalculations

SYSTEM STATE:

- High control-plane activity with increased CPU utilization and delayed routing convergence

CAUSAL RELATIONSHIPS:

[LSA Flooding] → [SPF Recalculation Overhead]
[LSA Flooding] → [High CPU Utilization]
[LSA Flooding] → [Control Plane Instability]
[LSA Flooding] → [Routing Update Delays]
[LSA Flooding] → [Traffic Rerouting]
[LSA Flooding] → [Packet Drops (Control/Data Plane)]
[LSA Flooding] → [Increased Latency / Jitter]
[LSA Flooding] → [Application Slowdown]
[LSA Flooding] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Routing instability

- Frequent SPF recalculations

- Control-plane overload

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Degraded application performance

• Operational Impact:

- Increased MTTR

- OSPF tuning (timers, throttling)

- Topology optimization effort

• Security Implications:

- Monitoring blind spots due to control-plane overload

- Reduced visibility during high LSA activity

- Potential exploitation via control-plane exhaustion

    - IS-IS: Adjacency Throttling Misconfiguration

MECHANISM:

- IS-IS adjacency throttling misconfiguration → Excessive or insufficient pacing of hello/adjacency formation causing instability and delayed convergence

SYSTEM STATE:

- Unstable IS-IS neighbor relationships with inconsistent route propagation and oscillating topology state

CAUSAL RELATIONSHIPS:

[IS-IS Throttling Misconfig] → [Adjacency Formation Delay/Flap]
[IS-IS Throttling Misconfig] → [Route Propagation Delay]
[IS-IS Throttling Misconfig] → [Traffic Rerouting]
[IS-IS Throttling Misconfig] → [Reduced Available Bandwidth]
[IS-IS Throttling Misconfig] → [High Utilization (≥80–100%)]
[IS-IS Throttling Misconfig] → [Queue Build-up]
[IS-IS Throttling Misconfig] → [Packet Drops (1–25%)]
[IS-IS Throttling Misconfig] → [Increased Latency / Jitter]
[IS-IS Throttling Misconfig] → [Application Instability]
[IS-IS Throttling Misconfig] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- IS-IS adjacency instability

- Delayed convergence

- Routing inconsistencies

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Intermittent connectivity

• Operational Impact:

- Increased MTTR

- Configuration tuning effort

- Protocol parameter validation

• Security Implications:

- Monitoring blind spots during instability

- Reduced visibility due to route oscillation

    - EIGRP: Stuck-in-Active (SIA) Condition

MECHANISM:

- EIGRP Stuck-in-Active (SIA) condition → Queries not answered within timeout causing route recomputation and adjacency resets

SYSTEM STATE:

- Delayed route convergence with unstable EIGRP neighbors and repeated topology recalculations

CAUSAL RELATIONSHIPS:

[SIA Condition] → [EIGRP Query Timeout]
[SIA Condition] → [Neighbor Reset]
[SIA Condition] → [Route Recalculation]
[SIA Condition] → [Traffic Rerouting]
[SIA Condition] → [Reduced Available Bandwidth]
[SIA Condition] → [High Utilization (≥80–100%)]
[SIA Condition] → [Queue Build-up]
[SIA Condition] → [Packet Drops (1–25%)]
[SIA Condition] → [Increased Latency / Jitter]
[SIA Condition] → [Application Instability]
[SIA Condition] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- EIGRP neighbor instability

- Repeated route recalculations

- Convergence delays

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Intermittent connectivity issues

• Operational Impact:

- Increased MTTR

- Topology optimization required

- Query boundary (stub) configuration effort

• Security Implications:

- Monitoring blind spots during convergence

- Reduced visibility due to route instability

    - Route-Map / Prefix-List Change

MECHANISM:

- Route-map or prefix-list modification → Change in route filtering or policy causing traffic redistribution and path shifts

SYSTEM STATE:

- Altered routing decisions leading to uneven load distribution and potential congestion on selected paths

CAUSAL RELATIONSHIPS:

[Policy Change] → [Route Filtering / Attribute Modification]
[Policy Change] → [Path Selection Change]
[Policy Change] → [Traffic Rerouting]
[Policy Change] → [Reduced Available Bandwidth]
[Policy Change] → [High Utilization (≥80–100%)]
[Policy Change] → [Queue Build-up]
[Policy Change] → [Packet Drops (1–25%)]
[Policy Change] → [Increased Latency / Jitter]
[Policy Change] → [Application Instability]
[Policy Change] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Routing policy-induced changes

- Path recalculation events

- Uneven traffic distribution

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Intermittent or degraded application performance

• Operational Impact:

- Increased MTTR

- Policy validation and rollback effort

- Coordination across network teams

• Security Implications:

- Unintended traffic exposure or blocking

- Monitoring inconsistencies due to path changes

- Reduced visibility during transition

    - Route-Policy / Filter Misconfiguration

MECHANISM:

- Incorrect route policy or filter configuration → Unintended route advertisement, suppression, or attribute modification causing traffic misdirection

SYSTEM STATE:

- Inconsistent routing behavior leading to suboptimal paths and uneven traffic distribution

CAUSAL RELATIONSHIPS:

[Policy Misconfiguration] → [Incorrect Route Advertisement/Suppression]
[Policy Misconfiguration] → [Suboptimal Path Selection]
[Policy Misconfiguration] → [Traffic Rerouting]
[Policy Misconfiguration] → [Reduced Available Bandwidth]
[Policy Misconfiguration] → [High Utilization (≥80–100%)]
[Policy Misconfiguration] → [Queue Build-up]
[Policy Misconfiguration] → [Packet Drops (1–25%)]
[Policy Misconfiguration] → [Increased Latency / Jitter]
[Policy Misconfiguration] → [Application Instability]
[Policy Misconfiguration] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Routing inconsistencies

- Path imbalance

- Unexpected convergence events

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Partial or degraded connectivity

• Operational Impact:

- Increased MTTR

- Policy validation and correction effort

- Cross-domain coordination

• Security Implications:

- Unintended exposure or blocking of routes

- Monitoring blind spots due to incorrect filtering

- Reduced visibility during misrouting

  - Hardware / Software Issues

    - Control Plane CPU Exhaustion

MECHANISM:

- Control plane CPU exhaustion → Inability to process routing protocols, control traffic, and management tasks under load

SYSTEM STATE:

- Overloaded control plane causing delayed or dropped protocol packets while data plane continues forwarding

CAUSAL RELATIONSHIPS:

[CPU Exhaustion] → [Control Plane Packet Drops]
[CPU Exhaustion] → [Routing Protocol Instability]
[CPU Exhaustion] → [BGP/OSPF/EIGRP Neighbor Flaps]
[CPU Exhaustion] → [Routing Update Delays]
[CPU Exhaustion] → [Traffic Rerouting]
[CPU Exhaustion] → [Packet Loss (Control/Data Plane)]
[CPU Exhaustion] → [Increased Latency / Jitter]
[CPU Exhaustion] → [Application Instability]
[CPU Exhaustion] → [Increased MTTR]
[CPU Exhaustion] → [SIEM Log Gaps]

IMPACT GROUPING:

• Network Stability:

- Control plane instability

- Routing adjacency flaps

- Delayed convergence

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Intermittent connectivity issues

• Operational Impact:

- Increased MTTR

- CPU optimization or hardware upgrade

- Control-plane policing (CoPP) tuning

• Security Implications:

- Log loss and monitoring gaps

- Increased susceptibility to control-plane attacks

- Reduced visibility during overload

    - Route Processor (RP) Failover

MECHANISM:

- Route Processor (RP) failover → Switchover between active and standby control plane causing temporary disruption in routing and control functions

SYSTEM STATE:

- Transient control-plane interruption with routing state resynchronization and traffic redistribution

CAUSAL RELATIONSHIPS:

[RP Failover] → [Control Plane Switchover]
[RP Failover] → [Routing Process Restart / Sync]
[RP Failover] → [Temporary Route Unavailability]
[RP Failover] → [Traffic Rerouting]
[RP Failover] → [Reduced Available Bandwidth]
[RP Failover] → [High Utilization (≥80–100%)]
[RP Failover] → [Queue Build-up]
[RP Failover] → [Packet Drops (1–25%)]
[RP Failover] → [Increased Latency / Jitter]
[RP Failover] → [Application Instability]
[RP Failover] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Control-plane switchover event

- Temporary routing instability

- Convergence and resynchronization

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Short-duration connectivity interruption

• Operational Impact:

- Increased MTTR

- Failover validation and redundancy checks

- Troubleshooting of failover triggers

• Security Implications:

- Monitoring blind spots during switchover

- Temporary loss of visibility and logging

    - FIB / RIB Sync Issues

MECHANISM:

- FIB/RIB synchronization issue → Inconsistency between control-plane routing table (RIB) and data-plane forwarding table (FIB)

SYSTEM STATE:

- Mismatch between routing decisions and actual packet forwarding leading to drops or misrouted traffic

CAUSAL RELATIONSHIPS:

[FIB/RIB Mismatch] → [Incorrect Forwarding Entries]
[FIB/RIB Mismatch] → [Packet Misrouting / Drops]
[FIB/RIB Mismatch] → [Traffic Rerouting (Inefficient)]
[FIB/RIB Mismatch] → [High Utilization (≥80–100%)]
[FIB/RIB Mismatch] → [Queue Build-up]
[FIB/RIB Mismatch] → [Packet Drops (1–30%)]
[FIB/RIB Mismatch] → [Increased Latency / Jitter]
[FIB/RIB Mismatch] → [Application Instability]
[FIB/RIB Mismatch] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Forwarding inconsistencies

- Routing vs forwarding mismatch

- Path instability

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Intermittent or incorrect connectivity

• Operational Impact:

- Increased MTTR

- Troubleshooting synchronization issues

- Potential process restart or device reload

• Security Implications:

- Monitoring blind spots due to incorrect forwarding

- Reduced visibility into actual traffic paths

- Potential policy enforcement gaps

    - TCAM / FIB Programming Delays

MECHANISM:

- TCAM / FIB programming delays → Slow installation of forwarding entries causing temporary forwarding gaps

SYSTEM STATE:

- Control-plane updates not immediately reflected in data-plane leading to transient packet drops or suboptimal forwarding

CAUSAL RELATIONSHIPS:

[Programming Delay] → [Delayed FIB Updates]
[Programming Delay] → [Forwarding Inconsistency]
[Programming Delay] → [Packet Drops (1–20%)]
[Programming Delay] → [Traffic Rerouting (Inefficient)]
[Programming Delay] → [High Utilization (≥80–100%)]
[Programming Delay] → [Queue Build-up]
[Programming Delay] → [Increased Latency / Jitter]
[Programming Delay] → [Application Instability]
[Programming Delay] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Temporary forwarding inconsistency

- Delay in route convergence at data-plane

- Path instability

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Intermittent connectivity issues

• Operational Impact:

- Increased MTTR

- Hardware/software tuning required

- Vendor support or firmware upgrade

• Security Implications:

- Monitoring blind spots during update delays

- Potential policy enforcement gaps

- Reduced visibility of transient traffic paths

    - Memory Leak in Routing Process

MECHANISM:

- Memory leak in routing process → Gradual depletion of memory resources causing control-plane degradation and instability

SYSTEM STATE:

- Increasing memory utilization leading to delayed routing updates, process restarts, or partial control-plane failure

CAUSAL RELATIONSHIPS:

[Memory Leak] → [High Memory Utilization]
[Memory Leak] → [Routing Process Slowdown]
[Memory Leak] → [Control Plane Instability]
[Memory Leak] → [Routing Update Delays]
[Memory Leak] → [Traffic Rerouting]
[Memory Leak] → [Packet Drops (Control/Data Plane)]
[Memory Leak] → [Increased Latency / Jitter]
[Memory Leak] → [Application Instability]
[Memory Leak] → [Increased MTTR]
[Memory Leak] → [SIEM Log Gaps]

IMPACT GROUPING:

• Network Stability:

- Routing instability

- Delayed convergence

- Control-plane degradation

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Intermittent connectivity

• Operational Impact:

- Increased MTTR

- Process restart or device reload required

- Software patch / bug fix effort

• Security Implications:

- Log loss and monitoring gaps

- Reduced visibility during degraded state

- Increased exposure to control-plane attacks

    - Line Card Crash / Reload

MECHANISM:

- Line card crash or reload → Sudden loss of interfaces on the affected module causing traffic redistribution

SYSTEM STATE:

- Partial device failure with reduced forwarding capacity and increased load on remaining line cards

CAUSAL RELATIONSHIPS:

[Line Card Failure] → [Interface Down Events]
[Line Card Failure] → [Traffic Rerouting]
[Line Card Failure] → [Reduced Available Bandwidth]
[Line Card Failure] → [High Utilization (≥80–100%)]
[Line Card Failure] → [Queue Build-up]
[Line Card Failure] → [Packet Drops (1–30%)]
[Line Card Failure] → [Increased Latency / Jitter]
[Line Card Failure] → [TCP Retransmissions]
[Line Card Failure] → [Application Instability]
[Line Card Failure] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Partial forwarding capacity loss

- Interface and path failover

- Congestion on remaining modules

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Degraded application performance

• Operational Impact:

- Increased MTTR

- Hardware troubleshooting / replacement

- Traffic redistribution planning

• Security Implications:

- Monitoring blind spots on affected interfaces

- Reduced visibility during module failure

    - Software Bug Causing Route Flapping

MECHANISM:

- Software defect causing route instability → Frequent route withdrawal and reinstallation leading to continuous traffic shifts

SYSTEM STATE:

- Persistent routing oscillation with unstable forwarding paths and repeated convergence events

CAUSAL RELATIONSHIPS:

[Route Flap Bug] → [Route Withdrawal / Reinstallation]
[Route Flap Bug] → [Frequent Convergence Events]
[Route Flap Bug] → [Traffic Rerouting]
[Route Flap Bug] → [Reduced Available Bandwidth]
[Route Flap Bug] → [High Utilization (≥80–100%)]
[Route Flap Bug] → [Queue Build-up]
[Route Flap Bug] → [Packet Drops (1–30%)]
[Route Flap Bug] → [Increased Latency / Jitter]
[Route Flap Bug] → [TCP Retransmissions]
[Route Flap Bug] → [Application Instability]
[Route Flap Bug] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Continuous route oscillation

- Frequent convergence events

- Path instability

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Intermittent or degraded connectivity

• Operational Impact:

- Increased MTTR

- Software debugging / patching required

- Vendor support engagement

• Security Implications:

- Monitoring blind spots during route instability

- Reduced visibility due to frequent path changes

- Potential exploitation of unstable routing behavior

    - NSF / SSO Failure

MECHANISM:

- NSF/SSO failure → Stateful switchover mechanism fails causing full control-plane reset and route loss

SYSTEM STATE:

- Loss of control-plane continuity leading to routing restart and temporary forwarding disruption

CAUSAL RELATIONSHIPS:

[NSF/SSO Failure] → [Control Plane Reset]
[NSF/SSO Failure] → [Routing Process Restart]
[NSF/SSO Failure] → [Route Withdrawal]
[NSF/SSO Failure] → [Traffic Rerouting]
[NSF/SSO Failure] → [Reduced Available Bandwidth]
[NSF/SSO Failure] → [High Utilization (≥80–100%)]
[NSF/SSO Failure] → [Queue Build-up]
[NSF/SSO Failure] → [Packet Drops (1–30%)]
[NSF/SSO Failure] → [Increased Latency / Jitter]
[NSF/SSO Failure] → [Application Instability]
[NSF/SSO Failure] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Loss of graceful failover

- Routing instability

- Convergence events

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Temporary connectivity interruption

• Operational Impact:

- Increased MTTR

- Failover validation required

- Software/hardware troubleshooting

• Security Implications:

- Monitoring blind spots during failover failure

- Temporary loss of logging and visibility

- Potential exposure during control-plane reset

  - External Factors

    - Upstream Provider BGP Session Drop

MECHANISM:

- Upstream provider BGP session drop → Loss of external route exchange causing traffic redistribution to alternate paths

SYSTEM STATE:

- External routing dependency failure leading to reduced path diversity and congestion on remaining routes

CAUSAL RELATIONSHIPS:

[BGP Session Drop] → [Route Withdrawal]
[BGP Session Drop] → [Traffic Rerouting]
[BGP Session Drop] → [Reduced Available Bandwidth]
[BGP Session Drop] → [High Utilization (≥80–100%)]
[BGP Session Drop] → [Queue Build-up]
[BGP Session Drop] → [Packet Drops (1–30%)]
[BGP Session Drop] → [Increased Latency / Jitter]
[BGP Session Drop] → [TCP Retransmissions]
[BGP Session Drop] → [Application Slowdown]
[BGP Session Drop] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- External routing instability

- Reduced path redundancy

- Convergence events

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Degraded external connectivity

• Operational Impact:

- Increased MTTR

- ISP coordination required

- Routing policy adjustments

• Security Implications:

- Monitoring blind spots for external routes

- Reduced visibility during session loss

    - Internet Exchange (IX) Outage

MECHANISM:

- Internet Exchange (IX) outage → Loss of peering point causing traffic to reroute via longer or limited transit paths

SYSTEM STATE:

- Reduced peering capacity with increased load on upstream transit links leading to congestion

CAUSAL RELATIONSHIPS:

[IX Outage] → [Peering Path Loss]
[IX Outage] → [Traffic Rerouting]
[IX Outage] → [Reduced Available Bandwidth]
[IX Outage] → [High Utilization (≥80–100%)]
[IX Outage] → [Queue Build-up]
[IX Outage] → [Packet Drops (1–30%)]
[IX Outage] → [Increased Latency / Jitter]
[IX Outage] → [TCP Retransmissions]
[IX Outage] → [Application Slowdown]
[IX Outage] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Loss of peering routes

- Increased dependency on transit paths

- Routing convergence events

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Degraded performance for internet services

• Operational Impact:

- Increased MTTR

- Coordination with IX / ISP providers

- Traffic engineering adjustments

• Security Implications:

- Monitoring blind spots due to rerouted traffic

- Reduced visibility across alternate paths

    - DDoS Attack Targeting Control Plane

MECHANISM:

- DDoS attack targeting control plane → Excessive control traffic overwhelms CPU and protocol processing capacity

SYSTEM STATE:

- Control plane saturation leading to delayed or dropped routing/control packets while data plane may partially function

CAUSAL RELATIONSHIPS:

[Control Plane DDoS] → [High CPU Utilization]
[Control Plane DDoS] → [Control Plane Packet Drops]
[Control Plane DDoS] → [Routing Protocol Instability]
[Control Plane DDoS] → [BGP/OSPF/EIGRP Neighbor Flaps]
[Control Plane DDoS] → [Routing Update Delays]
[Control Plane DDoS] → [Traffic Rerouting]
[Control Plane DDoS] → [Packet Loss (Control/Data Plane)]
[Control Plane DDoS] → [Increased Latency / Jitter]
[Control Plane DDoS] → [Application Instability]
[Control Plane DDoS] → [Increased MTTR]
[Control Plane DDoS] → [SIEM Log Gaps]

IMPACT GROUPING:

• Network Stability:

- Control plane instability

- Routing adjacency flaps

- Delayed convergence

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Intermittent connectivity issues

• Operational Impact:

- Increased MTTR

- Incident response and mitigation (CoPP, rate limiting)

- Vendor / ISP coordination

• Security Implications:

- Active attack condition (DoS)

- SIEM/log loss due to overload

- Intrusion detection bypass

- Reduced visibility during attack

    - DNS / NTP Failure (Influencing Timers)

MECHANISM:

- DNS/NTP failure affecting timers → Incorrect time synchronization or resolution delays impacting protocol timers and control-plane operations

SYSTEM STATE:

- Timer inconsistencies causing premature session expiry, delayed processing, or synchronization issues across network devices

CAUSAL RELATIONSHIPS:

[Timer Inconsistency] → [Protocol Timeout Mismatch]
[Timer Inconsistency] → [Neighbor Session Drops]
[Timer Inconsistency] → [Route Withdrawal / Reinstallation]
[Timer Inconsistency] → [Traffic Rerouting]
[Timer Inconsistency] → [Reduced Available Bandwidth]
[Timer Inconsistency] → [High Utilization (≥80–100%)]
[Timer Inconsistency] → [Queue Build-up]
[Timer Inconsistency] → [Packet Drops (1–25%)]
[Timer Inconsistency] → [Increased Latency / Jitter]
[Timer Inconsistency] → [Application Instability]
[Timer Inconsistency] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Protocol timer inconsistencies

- Session instability (BGP/OSPF/NTP sync issues)

- Increased convergence events

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Intermittent connectivity issues

• Operational Impact:

- Increased MTTR

- Time synchronization troubleshooting

- Dependency validation (DNS/NTP services)

• Security Implications:

- Log timestamp inaccuracies

- Monitoring and correlation issues

- Reduced effectiveness of forensic analysis

    - Cloud Provider Route Withdrawal

MECHANISM:

- Cloud provider route withdrawal → Loss of advertised prefixes causing traffic redirection to alternate or longer paths

SYSTEM STATE:

- External routing change leading to reduced path availability and increased load on remaining routes

CAUSAL RELATIONSHIPS:

[Route Withdrawal] → [Loss of Preferred Path]
[Route Withdrawal] → [Traffic Rerouting]
[Route Withdrawal] → [Reduced Available Bandwidth]
[Route Withdrawal] → [High Utilization (≥80–100%)]
[Route Withdrawal] → [Queue Build-up]
[Route Withdrawal] → [Packet Drops (1–30%)]
[Route Withdrawal] → [Increased Latency / Jitter]
[Route Withdrawal] → [TCP Retransmissions]
[Route Withdrawal] → [Application Slowdown]
[Route Withdrawal] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- External routing instability

- Path changes and convergence events

- Increased dependency on alternate routes

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Degraded access to cloud services

• Operational Impact:

- Increased MTTR

- Coordination with cloud provider

- Traffic engineering / failover adjustments

• Security Implications:

- Monitoring blind spots due to path changes

- Reduced visibility into rerouted traffic

- Potential exposure during routing instability

    - BGP Route Leak / Hijack

MECHANISM:

- BGP route leak or hijack → Incorrect or malicious route advertisement causing traffic redirection and path concentration

SYSTEM STATE:

- Suboptimal or unauthorized routing paths leading to congestion on unintended links

CAUSAL RELATIONSHIPS:

[Route Leak / Hijack] → [Suboptimal Path Selection]
[Route Leak / Hijack] → [Traffic Concentration]
[Route Leak / Hijack] → [Traffic Rerouting]
[Route Leak / Hijack] → [Reduced Available Bandwidth]
[Route Leak / Hijack] → [High Utilization (≥80–100%)]
[Route Leak / Hijack] → [Queue Build-up]
[Route Leak / Hijack] → [Packet Drops (1–30%)]
[Route Leak / Hijack] → [Increased Latency / Jitter]
[Route Leak / Hijack] → [TCP Retransmissions]
[Route Leak / Hijack] → [Application Instability]
[Route Leak / Hijack] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Routing instability

- Unexpected path changes

- Congestion on unintended links

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Partial or complete service inaccessibility

• Operational Impact:

- Increased MTTR

- Routing investigation and mitigation

- Coordination with upstream providers

• Security Implications:

- Potential traffic interception

- Monitoring blind spots due to path deviation

- Increased risk of data exposure

    - RPKI Validation Failure

MECHANISM:

- RPKI validation failure → Legitimate routes marked invalid and dropped due to ROA mismatch or validation issues

SYSTEM STATE:

- Reduced route availability leading to unintended path selection or traffic blackholing

CAUSAL RELATIONSHIPS:

[RPKI Validation Failure] → [Route Rejection]
[RPKI Validation Failure] → [Loss of Valid Prefixes]
[RPKI Validation Failure] → [Traffic Rerouting]
[RPKI Validation Failure] → [Reduced Available Bandwidth]
[RPKI Validation Failure] → [High Utilization (≥80–100%)]
[RPKI Validation Failure] → [Queue Build-up]
[RPKI Validation Failure] → [Packet Drops (1–30%)]
[RPKI Validation Failure] → [Increased Latency / Jitter]
[RPKI Validation Failure] → [Application Instability]
[RPKI Validation Failure] → [Increased MTTR]

IMPACT GROUPING:

• Network Stability:

- Routing inconsistencies

- Loss of expected paths

- Convergence events

• Service Disruption:

- Packet loss

- Increased latency / jitter

- Partial reachability issues

• Operational Impact:

- Increased MTTR

- RPKI/ROA validation troubleshooting

- Coordination with prefix owners/providers

• Security Implications:

- Over-filtering of legitimate routes

- Monitoring blind spots due to route rejection

- Reduced visibility into affected prefixes